## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Engineering & Technology

Subject: Applied Mechanics-I(2005) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the various prefixes and interconversion between the prefixes. |  |
| b) | What are scalars and vectors? |  |
| c) | Explain the different types of forces. |  |
| d) | Define ‘couple’ & give its unit |  |
| e) | State Varignon’s theorem and explain. |  |
| f) | What do you mean by equilibrium? State the various conditions of equilibrium. |  |
| g) | Define coefficient of friction and angle of friction. |  |
| h) | State the kinematic equations. |  |
| i) | With neat sketch locate the c.g. of square pyramid. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Four forces of 5N, 8N, 6N and 7N make an angle of 0°, 80°, 160° and 280° with respect to +ve x-axis in anticlockwise direction. Find out the resultant of the four forces. |  |
| b) | Three forces of magnitude 10KN, 15KN and 12KN are acting along the sides of an equilateral triangle of side 2m in clockwise direction. Determine the resultant and locate it with respect to any corner of the triangle. |  |
| c) | Find out the resistant of the system of forces shown in the fig.1 and locate the resultant with respect to point ‘A’. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | In a parallel force system, there are four forces of magnitude 8N acting upward, 12N acting upward, 10N acting downward and 4N acting downwards. The typical distance between two forces is 1.5m. Find the magnitude and direction of the resultant force and locate the resultant with respect to any force. |  |
| b) | Find out the tension and the force in the cable and beam. Refer fig.2. |  |
| c) | Calculate the tension in the cables. Refer fig.3. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The 8m beam is loaded as shown in fig.4. Calculate the reactions at the supports. |  |
| b) | Determine the reactions at the supports. Refer fig.5. |  |
| c) | Calculate the force ‘F’ necessary to raise the block in upward direction. Refer fig.6. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Determine the force ‘F’ necessary to apply on the wedge to lift a block of weight 20N. The coefficient of friction between the surfaces is 0.25. Refer fig.7. |  |
| b) | Derive the Kinematic equations. | …..2/- |
|  | -2- |  |
| c) | An electric train starts from station and comes to another station which is 5 Km from first on a straight track. For three quarters of distance, the train is uniformly accelerated and for the remaining uniformly retarded. If train takes 12 minutes to cover whole journey, find i) Max speed attained by the train ii) Acceleration of the train iii) Retardation of train. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A train starts from rest moves with uniform acceleration for first kilometer and attain the maximum velocity. With this velocity it moves for next 6 Km and then comes to rest with uniform retardation. If total journey is 9 Km and it takes 6 minutes for the train to complete it. Calculate the maximum velocity attained. |  |
| b) | Calculate the c.g. of the given lamina. Refer fig.8. |  |
| c) | i) Define Rigid and deformable body. ii) State characteristics of couple. |  |