## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Electronics Engg./EC/MED/EI

Subject: Basic Electronics Engineering(CS308) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define any three important specifications of a zener diode. |  |
| b) | Briefly comment on zener breakdown and avalanche breakdown mechanisms. |  |
| c) | Draw the structure of NPN transistor. Write the current equation for the same. |  |
| d) | Compare BJT with JFET. (any three points) |  |
| e) | Write a short note on random access memory. |  |
| f) | Comment on flash memories. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the operation of a zener diode with the help of neat circuit diagram. |  |
| b) | Describe how a zener diode functions as a voltage regulator for following conditions: i) variable input voltage supply and fixed load resistance ii) variable load resistance and fixed input voltage supply. |  |
| c) | Calculate i) output voltage ii) zener current iii) voltage drop across series resistance and iv) power dissipation in zener diode for a zener voltage regulator circuit given that supply voltage Vs=100V, zener breakdown voltage Vz=45V, series resistance Rs=1K and load resistance = 2K. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the circuit diagram and waveforms for a capacitor filter circuit associated with a rectifier. Compare this filter with a series inductor filter. |  |
| b) | With the help of a neat circuit diagram, sketch and explain the output characteristics of a transistor in common emitter configuration. |  |
| c) | Explain the functioning of a PNP transistor with a neat diagram. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Comment on leakage currents in a transistor. Also obtain the relationship between alpha and beta in a transistor. |  |
| b) | What do you understand by CE configuration in a transistor? Compare it with CB and CC configurations in detail. |  |
| c) | Draw the characteristics of depletion type MOSFET and explain. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the structural diagrams of n channel JFET and p channel JFET. Also sketch their symbols. Explain the three important JFET parameters in detail. |  |
| b) | Explain the operation of p channel JFET with the help of neat diagram. | …….2/- |
|  | -2- |  |
| c) | Draw the structure of enhancement type MOSFET and describe its constructional details. Also draw its symbol. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw neat diagram of CMOS NAND gate and explain its circuit operation in brief. |  |
| b) | What is meant by propagation delay with respect to digital IC logic families? Also comment on static and dynamic memories. |  |
| c) | Write short notes on: i) ROM ii) PROM. |  |

***********