## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Civil Engg./CT

Subject: Quantity Surveying & Costing(4014) [Old]

# Time Duration: 4 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List Attributes of ideal quantity Surveyor. |  |
| b) | Write factors affecting value of asset. |  |
| c) | What is material at site record? |  |
| d) | State unit of measurement for: i) Mangalore tile roofing ii) RCC grill iii) Half brick partition wall iv) Ceramic tile flooring v) Wood work in frame vi) Plastering. |  |
| e) | What do you mean by administrative approval and expenditure sanction? |  |
| f) | Define market value, salvage value, book value. |  |
| g) | Elaborate on deposit works. |  |
| Q.No.2. | Answer any one of the following Questions: | (15) |
| a) | Fig.1 shows plan and section of wall of single storeyed building. Calculate quantities of any five items. - 12mm thick Internal plaster - Tile flooring - Dadoing for Bath & WC upto 2.0m - Brickwork in superstructure - Wood work in door/window shutter - PCC (1:3:6) in foundation & plinth |  |
| b) | Determine quantities of earthwork for portion of road between stations P & Q for following data. One chain is 20m, width of road= 12m, side slope in cutting 1:1, side slope in banking 1.5:1. Draw longitudinal & cross sectional view. Lateral slope of ground may be assumed as level. Formation level of chainage is 120m & road has rising gradient of l in 150. |  |
| Chainage; 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; 10. Ground level in m; 121.1; 121.2; 120.9; 121.4; 120.6; 120.7; 120.6; 120.4; 119.1; 119.5; 119.7. |
| Q.No.3. | Answer the following Questions: | 3 x 5 = 15 |
| a) | Prepare rate analysis for 12mm thick plaster in cement mortar 1:6 quantity –10m2. |  |
| b) | Carry out rate analysis of RCC 1:1½:3 including centering & shuttering & excluding steel work for 10m3 quantity of work. |  |
| c) | Write detailed specification for 6mm thick cement plaster for ceiling. |  |
| Q.No.4. | Answer any three of the following Questions: | 3 x 5 = 15 |
| a) | Draft a tender notice inviting tender for construction of a Library building for the institute with estimated cost of Rs.1.5 crore to be completed in 2 years. |  |
| b) | Write short note on cash-imprest and temporary advance. |  |
| c) | What are characteristics of NIT? |  |
| d) | List down all methods of estimating approximate cost of building & explain any one in detail. | ……2/- |
|  | -2- |  |
| Q.No.5. | Write short notes on any three: | 3 x 5 = 15 |
| a) | Supplementary estimate |  |
| b) | Conditions of contract |  |
| c) | Contract document |  |
| d) | Advantages & disadvantages of item rate contract |  |