## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Electronics Engg./EC/EI/MED/COMP/E&EE

### Subject: Digital Electronics-II(4138) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw the logic circuit and write the truth table for half subtractor. |  |
| b) | Draw the logic diagram of 4:1 multiplexer. |  |
| c) | Define the following terms with respect to a D to A converter: (i) Accuracy (ii) Resolution. |  |
| d) | List the uses of PLD. |  |
| e) | Differentiate between ROM & EPROM. |  |
| f) | Draw a circuit for contact debouncing & explain the same. |  |
| g) | Explain why matrix addressing is better than memory addressing method. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and explain the working of Dual slope method of A to D conversion. |  |
| b) | Explain the working of a Full Adder circuit. |  |
| c) | Explain the operation of a 3 bit Binary adder-subtractor circuit. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the working of an encoder? What is a priority encoder? |  |
| b) | Explain the operation of a nibble multiplexer. |  |
| c) | With a logic diagram and truth table, explain the working of a 16 to 1 multiplexer. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain Weighted Resistor method of D to A conversion. |  |
| b) | Explain Linear Ramp method of A to D conversion. |  |
| c) | Explain Counter Type method of A to D conversion. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Obtain a 1024x8 memory using 256x8 memory chips. |  |
| b) | Draw a circuit of Astable oscillator using IC 555. Determine the frequency of oscillation, given RA=RB=1KΩ and C=0.1µF. Also calculate the duty cycle. |  |
| c) | What is an OR matrix? Draw & explain a typical OR matrix. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the operation of TTL clock circuit with the help of neat diagram. |  |
| b) | Draw a MOS RAM cell & explain its working. |  |
| c) | Explain the architecture of a PAL using neat diagram. |  |

***********