## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Theory of Structures-I(4022)/(FE601) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | For tension to develop what is the limit of eccentricity for a solid circular section? |  |
| b) | What are the uses of statically indeterminate structures? Explain with examples. |  |
| c) | What are the uses of Influence line diagram? |  |
| d) | Explain Principal planes and Principal stresses with neat sketches. |  |
| e) | How to find core of section for rectangular section, which is eccentrically loaded? |  |
| f) | Determine degree of indeterminancy for portal frame with fixed supports and King post truss supported on one roller and other hinge support. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A solid rectangular column 20cm wide and 15cm thick is carrying a vertical load of 10KN at an eccentricity of 5cm in a plane bisecting the thickness. Determine the maximum and minimum intensities of stress in the section. |  |
| b) | A steel pillar of circular cross section is of external diameter 40cm and metal thickness 4cm. A projecting bracket attached to it carries a load of 10KN. The load line being off the axis of the pillar by 25cm. Compute the maximum and minimum intensities of stress in the section. |  |
| c) | A column 3m x 4m supports a vertical load of 600KN at a point with eccentricity for vertical centroidal axis of 1m and eccentricity of 1.2m from horizontal centroidal axis. Find stresses at the corners of the column. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A rectangular element in a strained material is subjected to tensile stresses of 120 N/mm2 and 60 N/mm2 on mutually perpendicular planes together with a shear stress of 70 N/mm2. Find the principal stresses, principal planes and maximum shear in the block. |  |
| b) | The principal stresses at a point in a bar are 200 N/mm2 (tensile) and 100 N/mm2 (compressive). Determine the resultant stress in magnitude and direction on a plane inclined at 60° to the axis of the major principal stress. Also determine the maximum intensity of shear stress in the material at the point. |  |
| c) | A rectangular steel bar is subjected to a tensile stress of 100 N/mm2 as well as a shear stress of 40N/mm2. Determine the principal stresses and the principal planes. |  |
|  |  | …….2/- |
|  | -2- |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A simply supported girder has a span of 12m. A 20 KN wheel load moves from one end to the other end on the span of the girder. Find the maximum bending moment, which can occur at a section 4m from the left end. Use Influence line diagram. |  |
| b) | Two wheel loads 8KN and 20KN spaced at 2m apart move along the span of the girder of span 16m. Find the maximum bending moment that can occur at a section 6m for the left end. Any wheel load can load the other. Use Influence line diagram. |  |
| c) | For simply supported beam of span L and draw ILD for (i) bending moment at any section at ‘a’ from left support (ii) shear force at any section ‘a’ from left support. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | i) State torsion equation for circular shafts and list the parameters involved. ii) State the assumption made in the theory of pure torsion. |  |
| b) | A hollow circular shaft 20mm thick transmits 300KW at 200 rpm. Determine the external diameter of the shaft if the shear strain due to torsion is not to exceed 0.00086. Modulus of rigidity is 8 x 104 N/mm2. |  |
| c) | Find the power that can be transmitted by a shaft of 60mm diameter at a speed of 100 rpm if permissible shear stress is 50 N/mm2. |  |
| Q.No.6. | Answer the following Questions: |  |
| a) | Explain with examples how internal and external degree of indeterminancy is determined. What are the uses of indeterminate structures? | (6) |
|  | For students of Old scheme only: |  |
| b) | A flitched beam consists of two wooden joists 100mm wide and 200mm deep with a steel plate 140mm deep and 10mm thick place symmetrically between them. If the maximum stress in the wooden joist is 7 N/mm2, find the corresponding maximum stress reached in steel. Also find the moment of resistance of the section. Take Es = 20Ew | (6) |
|  | ### OR |  |
| b) | A timber beam 150mm wide and 200mm deep is to be reinforced by two steel plates each 150mm by 12.5mm in section placed symmetrically at the sides. Stress in timber is 6 N/mm2. Find the maximum stress in steel and moment of resistance of the section. Es = 20Ew | (6) |
|  | For students of Revised scheme only: |  |
| b) | A pipe of 500mm internal diameter and 100mm thickness contains a fluid at a pressure at 6 N/mm2. Find the maximum and minimum hoop stresses across the section. | (6) |
|  | ### OR |  |
| b) | A cylindrical shell 800mm in diameter and 10mm thickness is subjected to internal pressure of 2.5 N/mm2. Determine the hoop stress, longitudinal stress, circumferential strain and longitudinal strain. E = 20 x 105 N/mm2 and Poisson’s ratio is 0.25 | (6) |

***********