## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Electrical Engg./FT&EE

Subject: Elements of Civil Engineering(CS311) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Write the secondary classification of surveying. |  |
| b) | Distinguish between whole circle bearing system and quadrantal bearing system. |  |
| c) | Define i) Level line ii) Horizontal line iii) Bench mark. |  |
| d) | Explain different types of piles. |  |
| e) | Draw the line diagram for i) Fink ii) Pratt iii) Howe. |  |
| f) | Write any 3 components of building with their functions. |  |
| g) | What does plan, elevation and section indicates in a building? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is surveying? Write the general principle of surveying. |  |
| b) | The following are the bearings of a closed traverse SIDE; FB; BB. AB; N4530E; S4530W. BC; S4530E; N4530W. CD; S600W; N600E. DA; N7545W; S7545E. Calculate the interior angles of the traverse. |  |
| c) | Which are the temporary adjustments of dumpy level? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Following consecutive readings were taken with a dumpy level along a chain line at a common interval of 10m. The first reading was at chainage of 160m where RL is 100.500m. The instrument was shifted after 4th and 9th reading. 3.140, 2.235, 1.195, 0.850, 3.120, 2.650, 1.850, 1.460, 1.955, 1.230, 2.385 and 3.045. Find RL of all the points by HI method. |  |
| b) | Explain the procedure for measurement of vertical angle with proper sketch. |  |
| c) | Write any six parts of transit theodolite and their functions. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What do you mean by bearing capacity of soil? Distinguish between ultimate bearing capacity and safe bearing capacity of soil. |  |
| b) | With the aid of neat sketch explain the following: - Raft foundation ii) Grillage foundation |  |
| c) | Write down the classification of roof. Explain any two roof coverings. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What do you mean by fire resisting construction? List out various fire protection measures taken in a multi-storied building. |  |
| b) | Write down the ingredients of concrete. Which are the different grades of concrete used in construction work? Also write their functions. | ……2/- |
|  | -2- |  |
| c) | Explain the procedure from concreting till curing of a slab on 3rd floor level by using rotating drum mixer. |  |
| Q.No.6. | Following are the construction details of a single room with attached toilet. Size of room - 4.5m x 5.20m, attached toilet of 2.5m2 Height of roof above floor 3.0m, plinth level from ground = 0.6m, door - 1.2m x 2.10m, window - 1.20 x 1.50m, ventilator 0.6m x 0.6m, parapet wall 0.20m thick and 0.9m height above roof wall thickness – 0.23m roof – RCC slab 120mm thick chajja protection 0.6m from face of wall roof protection around 0.5m alround the room Draw the plan and section passing through the window opening. Use 1:50 scale. | (12) |

***********