## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Electronics Engg./E/EC/EI/E&EE/COMP/MED

Subject: Digital Electronics(CS309) [Revised]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Convert the following: i) 110111(2) =__________________(16) ii) FA.B (16) =___________________(10) iii) 110(10) =____________________(BCD) |  |
| b) | Convert 25(10) to Gray. |  |
| c) | Draw X-OR gate. Write down its Truth Table and Boolean equation for output. |  |
| d) | Explain Duality theorem with the help of two Boolean equations, which support the theorem. |  |
| e) | Draw symbols of T-FF and write down the Truth Table. |  |
| f) | What is a Register? What are the different types of Shift Registers? |  |
| g) | What is modulus of a counter? What will be the modulus of 4-bit Asynchronous up counter? |  |
| h) | What is a multiplexer? Draw block diagram of 4 to 1 Multiplexer. |  |
| Q.No.2. | Answer any three of the following. | 3 x 4 = 12 |
| a) | Draw logic diagram for Full Adder and write down its truth table. |  |
| b) | Simplify following using K-Map Y= m (1, 3, 5, 7, 8, 9) |  |
| c) | What is a Demultiplexer? Draw the block diagram and logic diagram of 1 to 4 Demultiplexer. |  |
| d) | Implement OR, AND, NOT using NOR. |  |
| e) | Draw 4 bit parallel binary Adder & explain with example. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw R-S Flip Flop using NAND gate. Write down its Truth Table. Convert it to D Flip Flop and write down truth table of D Flip Flop. |  |
| b) | Draw J-K Flip Flop. Write down the truth table and explain Master Slave Flip Flop. |  |
| c) | Perform the following using 2’s complement method: i) 15(10) - 25(10) ii) -15(10) + 25(10) iii) -15(10) - 25(10) |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is a counter? Draw 4bit Asynchronous down counter. Draw timing diagram and write down the count. |  |
| b) | Explain serial in parallel out shift register with block diagram and logic diagram with the data 1001. |  |
| c) | Write down any three points of differentiation between Synchronous and Asynchronous counters. |  |
|  |  | …….2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State different types of Analog to Digital converters. Explains Successive Approximation ADC. Why is the name Successive Approximation method? |  |
| b) | State different types of Digital to Analog converters. Explain Binary ladder DAC. Why is the name Binary ladder? |  |
| c) | Write a short note on: i) De Morgans theorems ii) ASCII code. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Prove that the following combinations are logically equal. |  |
| b) | Write a short note on ‘Decade Counter’. |  |
| c) | Write a short note on ‘Advantages of Digital signals over Analog’. |  |
| d) | Write the output of following Boolean equations: i) A+0= ii) A+1= iii) A⋅0= iv) A⋅A= |  |

***********