## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2016 Examinations

Programme: Electrical Engg./E&EE

Subject: Electrical Drives(4115)/(EE601) [Old/Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Give classification of Electric drives. State example of each type. |  |
| b) | What are passive loads? Give one example. |  |
| c) | What is most inefficient method of Electric braking? Why? |  |
| d) | State different types of enclosures used for motors. |  |
| e) | Which motor is preferred for lathe? Why? |  |
| f) | What is float switch? State its any two applications. |  |
| g) | Briefly explain how speed of three phase induction motor can be varied by cascade connection. |  |
| h) | State types of motors used in paper mills. (Only for students of old scheme) |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram explain four-quadrant operation. |  |
| b) | What do you mean by steady state stability? With the help of characteristics explain any one stable and any one unstable system. |  |
| c) | Briefly explain how load torque and moment of inertia is referred to motor shaft. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram and characteristics explain plugging of DC series motor. |  |
| b) | With the help of circuit diagram & characteristics explain rheostatic braking of d.c. shunt motor. |  |
| c) | For three phase induction motor explain speed control method by varying frequency. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat diagram explain for electric motor: - Continuous duty with intermittent periodic loading - Intermittent periodic duty with starting |  |
| b) | Briefly explain need of load equalisation. Also explain how it is achieved. |  |
| c) | An induction motor has to perform following duty cycle: - 75 KW for 10 minutes - No load for 5 minutes - 45 KW for 8 minutes - No load for 4 minutes |  |
|  | The duty cycle is repeated indefinitely. Determine a suitable capacity of continuous rated motor. Motors of standard continuous ratings of 35 KW, 45 KW. 55 KW & 65 KW are available. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat diagram explain working of a conductor. | …..2/- |
|  | -2- |  |
| b) | With the help of control diagram explain sequence control of two motors with time delay. |  |
| c) | With the help of power circuit diagram & control circuit diagram explain remote operation of motor. |  |
| Q.No.6. | Write short notes on any two: | 2 x 6 = 12 |
| a) | Rotor voltage injection method of speed control (Schrage motor) |  |
| b) | Drives used for pumps & lifts |  |
| c) | Interlocking control for motors |  |

***********