## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Mechanical Engg.

Subject: Energy Conversion(ME405) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | With reference to air compressors, define the following terms: i) Clearance volume ii) Volumetric efficiency iii) Free air delivered. |  |
| b) | State three important differences between S.I. engines and C.I. engines. |  |
| c) | Explain the difference between supercharging and turbocharging. State 2 advantages of turbocharged engine and over naturally aspirated engine. |  |
| d) | State three important features of Gas Turbine power plant. |  |
| e) | Explain the working of a solar still. |  |
| f) | State any three methods of converting wave energy to electricity. |  |
| g) | State any three maintenance issues as regard to Bio-gas plant. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the construction end working of a reciprocating air compressor. |  |
| b) | With the help of a neat sketch, explain the construction and working of a vane compressor. |  |
| c) | Draw a P-V diagram for a 2-stage reciprocating air compressor without clearance volume and state the condition for maximum efficiency. State four advantages of multistaging. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | In an engine working on ideal otto cycle, the temperature at the beginning and end of compression are 50C and 373C. Find the compression ratio and the air standard efficiency of the engine. |  |
| b) | With the help of a neat sketch, explain the working of a 4-stroke cycle Diesel engine. |  |
| c) | Draw a neat labelled sketch of a hydro-electric power plant and explain its working. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A gasoline engine working on four stroke develops a brake power of 20.9KW. A Morse test was conducted on this engine and the brake power(KW) obtained when each cylinder was made inoperative by short circuiting the spark plug are 14.9, 14.3, 14.8 and 14.5 respectively. The test was conducted at constant speed. Find its indicated power and mechanical efficiency. |  |
| b) | With the help of neat sketch, explain the construction and working of water cooling system in I.C. engines. |  |
| c) | Draw a neat labelled sketch of a nuclear power plant and explain its woking. | ……2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat labelled sketch of a vertical axis wind Turbine and state its advantages and disadvantages. |  |
| b) | Draw a neat labelled sketch of a solar water heater and explain its working. |  |
| c) | Draw a neat labelled sketch of a Bio-Gas plant and explain its working. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Write the classification of air compressors. State industrial application of each type. |  |
| b) | State four advantages of 4-stroke engine over 2-stroke engine. |  |
| c) | Draw a neat labelled sketch of a pressure feed lubrication system of an I.C. engine. |  |
| d) | State four important considerations in site selection of a wind power plant. |  |
| e) | Draw a neat labelled sketch of a solar cooker. |  |

***********