## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Fabrication Technology & Erection Engg./C/CT

Subject: Design of Steel Structures(FE701)/Design of Structures-I(Steel)[4024]

[Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of IS 800-1984 code and steel tables is permissible.

| Q.No.1. | Answer any three of the following Questions: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Draw a neat sketch of a plate girder and explain in brief the function of its different components. |  |
| b) | Explain the terms (1) effective length (2) least radius of gyration (3) slenderness ratio. |  |
| c) | Distinguish between web buckling and web crippling in beams. |  |
| d) | State the advantages and disadvantages of using tabular sections in trusses. |  |
| e) | Draw a neat sketch of a fillet weld and explain its salient features. |  |
|  | Only for students of revised scheme: |  |
| f) | State merits and demerits of bolted connections. |  |
| g) | Draw the stress-strain relationship for Mild steel and High tensile steel and mention salient features. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Design a lap joint to connect two plates 150 x 10mm and 150 x 8mm by fillet welds. Take permissible stress in tension equal to 150MPa and permissible stress in weld equal to 110MPa. |  |
| b) | Design a fillet weld joint for connecting two angle sections ISA 90x90x8mm on either side of a gusset plate to carry a tensile load of 250KN. Design the joint if size of weld is 6mm and allowable shear stress is 110MPa. |  |
| c) | A 150x16mm plate is welded to another plate by two side welds 120mm each and end fillet of 100mm length. Find the safe axial load to which this joint may be subjected if the size of the weld is 7mm. |  |
| d) | Describe the design procedure of bolted connection subject to the moment in the plane of bolts. (Only for students of revised scheme) |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A tension member is subjected to a pull of 250KN. Design suitable section if i) it consists of a single angle connected by one leg ii) it consists of double angle on either side of gusset plate and tacked along the length. |  |
| b) | The main tie member of a welded Firk truss has to carry a load of 160KN. Design a suitable section of member. |  |
| c) | Find the safe load carried by tension member consisting of two angles 50x50x6mm placed back to back and connected on opposite side of 8mm thick gusset. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Design a suitable rolled steel section to carry a load of 600KN. The height of column is 5m. Assume both ends pinned. |  |
| b) | Design a slab base for a column ISHB 350 @ 72.40 Kg/m carrying a load of 900KN. The column is supported on concrete pedestal having bearing capacity of 4 MPa. | …….2/- |
|  | -2- |  |
| c) | A compression member consists of two angles 100x65x8mm back to back and connected on opposite sides of gusset 10mm thick and has effective length of 1.0m. Find the safe load carried by the member. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A beam of 6 m span is required to carry a load of 40KN/m. Design a suitable section of the beam. Carryout check for deflection and shear for the beam section adopted. |  |
| b) | A beam of section ISWB 500 @ 95.2 Kg/m is simply supported over a span of 6.0m. The beam is laterally supported. Determine the safe load the beam can carry. Take fy = 250 N/mm2 and E = 2x105 N/mm2 |  |
| c) | Calculate the maximum value of a mid span concentrated load W that can be safely applied on a laterally supported beam of length 3.0m. The beam is simply supported at either end. The beam section is ISMB 250 and the load is applied at the top flange at midspan. Take fy = 250 MPa |  |
| Q.No.6. | Answer the following Questions: |  |
| a) | i) How is the spacing of purlins fixed? | (2) |
|  | ii) Why is it necessary to provide connections that will allow movement in the supports of trusses? | (2) |
|  | OR |  |
| b) | i) State the advantages of North light roof trusses over other forms of trusses. | (2) |
|  | ii) Mention various loads and their combinations considered in the design of Roof trusses. | (2) |
| c) | Design a channel purlin for an industrial building situated in Goa to support a 18 gauge galvanized corrugated iron sheet roof (with a dead weight of 131 N/m2) with the following data. Spacing of the trusses c/c = 5.0m Span of the truss = 15m Spacing of purlin c/c = 1.5m Intensity of wind pressure = 2.3 KN/m2 Yield stress of steel = 250 MPa | (8) |

***********