## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Electrical Engg./E&EE

Subject: Electrical Measurements & Instruments(EE303)/(4103)

[Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | A precise instrument need not be accurate. State true or false and justify. |  |
| b) | Explain importance of damping in indicating instruments. |  |
| c) | What is energy meter? Give its applications. |  |
| d) | Explain principle of working of ohmmeter. |  |
| e) | State principle of operation of frequency meter. |  |
| f) | What is the effect of control torque on the scale of indicating instrument? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Classify errors in measuring instrument. How can they be minimized? |  |
| b) | State and explain the various types of electric effect on basis of which measuring instruments are classified. |  |
| c) | Explain construction and principle of operation of PMMC instrument. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a note on errors in moving iron instruments. |  |
| b) | Compare PMMC and MI instruments. |  |
| c) | Write a note on “Three phase energy meter”. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram explain the construction and principle of operation of electrodynamometer type of wattmeter. |  |
| b) | With a neat circuit and phasor diagram explain measurement of power in 3 phase 3 wire circuit using two wattmeters. |  |
| c) | Two wattmeters connected to measure input to a balanced 3 phase circuit indicate 5KW and 0.5KW respectively. Find the power and power factor of the circuit when - Both readings are positive - One of the meters has been reversed. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a note on measurement of low resistance using Kelvin’s Bridge. |  |
| b) | Explain Ammeter-voltmeter method of measurement of resistance. |  |
| c) | Explain different types of errors in single phase induction type energy meter. (only for students of old scheme) |  |
| c) | Explain the concept and block diagram of Electronics energy meter. (only for students of revised scheme) |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the extension of range by shunt and multiplier. |  |
| b) | Explain measurement of power in 3 phase 3 wire circuit by 2 wattmeter method in conjunction with CT and PT. |  |
| c) | Write a note on phase sequence indicator. |  |
| d) | With a neat diagram explain the working of earth tester. |  |

***********