## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Electronics Engg./EC/EI/COMP/MED

Subject: Microprocessors(PD&I)-II(4147) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List down any three features of RS.232 bus standard. |  |
| b) | Explain functions of any three handshaking signal of 8255. |  |
| c) | Name the various timers modes of 8253. |  |
| d) | Explain functions of following signals of 8251 USART: - RxRDY ii) TxE iii) C/D |  |
| e) | Describe the AUTOLOAD feature of 8257 DMA controller. |  |
| f) | Explain the ICW3 control words for master 8259 and slave 8259. |  |
| g) | Describe the special fully nested mode in 8259 priority interrupt controller. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the timer modes of 8155 with suitable waveforms. |  |
| b) | Write an 8085 assembly language program to output the data byte AB H on port A of 8155. Assume that the address of port A is 81 H. Illustrate the control word configuration to be used. |  |
| c) | Describe the BSR control word format of 8255. Write an 8085 assembly language program to generate a square waveform using 8255. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and explain the block diagram of 8253. Describe the main features of 8253. |  |
| b) | Draw and explain the block diagram of 8259 priority interrupt controller. |  |
| c) | Describe the steps in the interrupt operation using a single 8259. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the different technologies used in inkjet printers? Describe working of an inkjet printer. |  |
| b) | Explain any two types of key switches with suitable diagrams. |  |
| c) | With a suitable block diagram, explain how characters are displayed on a monochrome CRT display. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is the GPIB bus standard used for? Explain the function of the bus management signals in GPIB. |  |
| b) | Describe the mode word format of 8251 in asynchronous & synchronous modes. |  |
| c) | Explain functions of following signals of 8257 in master mode & slave mode: - IOR ii) MEMW iii) CS |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe working of a temperature controller with the aid of a suitable diagram & assembly language program. |  |
| b) | Describe in detail the various modes in keyboard section of 8279. |  |
| c) | Explain the main features of display section of 8279. Differentiate between right-entry and left–entry modes of display section. |  |

***********