## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Theory of Structures-I(FE601)/(4022) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | With an example explain statically indeterminate structure. |  |
| b) | A hallow circular column of internal diameter (d), external diameter (D) has to support a non axial load (W) acting at a distance from centre of column. Find the value of (e) for no tension developed in the section. |  |
| c) | What are influence lines? Enlist the applications. |  |
| d) | Enlist the assumption in theory of pure torsion. |  |
| e) | Explain the concept of rolling loads with examples. |  |
| f) | Explain the following: i) Thin cylindrical shells ii) Rivetted shells. (Only for students of revised scheme) |  |
| f) | Explain the concept of flitched beam. (Only for students of old scheme) |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A column 75mm x 75mm has a circular cavity 37.5mm in diameter. Find the maximum values of load that can be applied at an eccentricity of 12 mm with respect to XX axis in order that maximum compressive stress may reach 80N/mm2. |  |
| b) | A concrete block has the cross-section of 2m x 2m. The block weighs 90KN and carries a vertically downward load of 20KN at point P on axis XX eccentric about YY axis. If the pressure under the block along edge AD is just twice the pressure under edge BC, find these pressures. |  |
| c) | A column section of 300mm external diameter and 150mm internal diameter supports an axial load of 2000KN and eccentric load P KN at an eccentricity of 300mm. If the compressive stress and tensile stress are not exceeding 140N/mm2 and 60N/mm2 respectively, find the magnitude of load P. |  |
| d) | A flitched beam consist of a wooden joist of 150mm wide and 300mm deep strengthen by a steel plate 12mm thick, 300mm deep one on either side of joist. If the maximum stress in wooden joist is 7N/mm2, find the corresponding maximum stress attained in steel. Also find the moment of resistance. ES=20EW. (Only for students of old scheme) |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | At a point in the web of a girder the bending stress is 60N/mm2 (tensile) and the shearing stress at the same point is 30N/mm2. Calculate: i) Principal stresses ii) Maximum shear stress. |  |
| b) | The principal stresses at a point in a bar are 250N/mm2 (tensile) and 100N/mm2 (Compressive). Determine the resultant stress in magnitude and direction on a plane inclined at 40° to the axis of major principal stress. Also determine the maximum intensity of shear stress in the material at the point. | …..2/- |
|  | -2- |  |
| c) | At a point in a bracket the stresses on two mutually perpendicular planes are 400N/mm2 tensile and 300N/mm2 tensile. The shear stress across these planes is 200N/mm2. Find the principal stresses and maximum shear stress at a point. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A uniform load of 2500N/m, 4m long crosses a girder of 20m span from left to right. Calculate the maximum SF and BM at a section of 6m from left hand support. |  |
| b) | Two wheel loads of 40KN and 80KN spaced at 4m apart roll on a girder of span 30m with either load leading the other. Find the maximum bending moment at a section 12m from left support. |  |
| c) | A simply support girder has a span of 10m. A 30KN wheel load moves from one end to other end on the span of girder. Find the maximum bending moment, which can occur at a section 3m from left end. Use Influence line diagram. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define the following and explain with expression: i) Torque ii) Power transmitted iii) Polar modulus. |  |
| b) | A hallow shaft is to transmit 250KW at 120rpm. If the shear stress is not to exceed 60MPa and internal diameter is 0.6 times the external diameter, find the diameter of the shaft. |  |
| c) | A circular shaft of 60mm diameter is running at 150rpm. If the shear stress not to exceed 50MPa, find the power which can be transmitted. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the following with examples: i) Internal and external stability of structure ii) Degrees of indeterminacy iii) Degrees of Freedom |  |
|  | Only for students of revised scheme: |  |
| b) | A thin cylindrical drum 1000mm in diameter 3m long has a shell thickness of 10mm. If the drum is subjected to an internal pressure of 3N/mm2, find i) Hoop stress ii) Longitudinal stress. |  |
| c) | A pipe of 600mm internal diameter and 80mm thickness contains a fluid at a pressure of 8N/mm2. Find the maximum and minimum hoop stress across the section. |  |
|  | Only for students of old scheme: |  |
| b) | The Principal stresses at a point in a bar are 260N/mm2(tensile) and 130N/mm2(Compressive). Find the resultant stress in magnitude and the direction on a plane inclined at 60° to the axis of major principal stress. Use Mohr’s circle method. |  |
| c) | In a strained material at a certain point the principal stresses are 120N/mm2 and 60N/mm2 both tensile. Find the normal tangential and resultant stresses across the plane though the point at 55° to the major principal plane. Use Mohr’s circle method. |  |

***********