## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Mining Engg.

Subject: Mine Surveying(MN611) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Enumerate uniqueness of mine surveying. |  |
| b) | Define error and write classification of errors. |  |
| c) | Enumerate GPS components and function of each component. |  |
| d) | Write volume formula for any three: i) Right circular cylinder ii) Frustum of pyramid iii) Sphere iv) Triangular prism (wedge). |  |
| e) | State purposes of curve setting in mines. |  |
| f) | List out equipments/instruments used in preparation of mine plans and sections. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain appointment and qualifications of mine surveyor. |  |
| b) | Define GPS and explain GPS and DGPS application in mining. |  |
| c) | Explain GIS principle of operation and functions. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain setting and observation process of total station. |  |
| b) | Two seams are 40 metres vertically apart dips 1 vertical in 7 horizontal (1 in 7). Calculate the length of cross measure drift if i) Drift is level ii) Rising 1 in 10 towards the dip of seam iii) Dips 1 in 2 in the direction of the seam. |  |
| c) | A mine reservoir is 35m long and 25m wide at the bottom, 5m is perpendicular height to the surface of water. When full the side are slope at angle of 45. How many litres of water it contains when full? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe in details how you would lay out a curve of 30m radius to join two underground roadways at right angle to other. |  |
| b) | Describe the method of correlation survey when two shafts are available. |  |
| c) | Explain stope survey in cut and fill stope. |  |
| Q.No.5. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Compute the cubic content of length of an embankment for which cross sectional area has been found as follows. |  |
| Distance(m); 0; 20; 40; 60; 80; 100; 120. Area (m2); 130; 195; 485; 545; 790; 345; 720. |
|  | ### OR |  |
| a) | Explain methods of calculating volume and weight of coal. |  |
| b) | Explain preparation and preservation of mine plans and sections. |  |
| Q.No.6. | Write short notes on: | 3 x 4 = 12 |
| a) | Mine survey office |  |
| b) | Instruments used in stope survey and correlation survey |  |
| c) | Gyrotheodolite |  |

***********