## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Electrical Engg./E&EE

Subject: Transformers(4106) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Give any three points of comparison between core type and shell type transformer. |  |
| b) | What is voltage regulation? Write expression for up regulation and down regulation. |  |
| c) | List the different losses occurring in a transformer. |  |
| d) | Explain the working of transformers on No-load. |  |
| e) | Discuss any three conditions necessary for parallel operation of three phase transformers. |  |
| f) | State any three advantages of computing quantities in per unit. |  |
| g) | Explain in brief how welding transformers are different from other transformers. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram explain working principle of transformers. |  |
| b) | Explain the functions of the following: i) Conservator tank ii) Tap changer iii) Temperature indicator |  |
| c) | What are the different types of transformer windings? Explain any one in detail with a diagram. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat phasor diagram explain working of a transformer on load for Inductive load. |  |
| b) | Draw and explain equivalent circuit of transformer referred to secondary side. |  |
| c) | A 30KVA, 2400/120V, 50Hz transformer has a primary winding resistance of 0.1Ω and leakage reactance of 0.22Ω. The secondary winding resistance is 0.035Ω and the leakage reactance is 0.012 Ω. Find the equivalent winding resistance, reactance and impedance referred to secondary side. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Derive the condition for maximum efficiency of a transformer. |  |
| b) | A 11000/230V, 150KVA, 1 phase, 50HZ transformer has core loss of 1.4KW and full load copper loss of 1.6KW. Determine i) The KVA load for maximum efficiency and value of maximum efficiency at unity p.f. ii) The efficiency at half full load 0.8 p.f. leading. |  |
| c) | Find the all day efficiency of 500KVA distribution transformer whose copper loss and iron loss at full load are 4.5 KW and 3.5 KW respectively. During a day of 24 hrs, it is loaded as under. No. of hours; Loading in KW; Power factor. 6; 400; 0.8. 10; 300; 0.75. 4; 100; 0.8. 4; 0; -. | ……2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain vector groups of transformer. |  |
| b) | Explain with a neat circuit diagram and vector diagram the effect of connection of two transformers belonging to different vector groups in parallel. |  |
| c) | Two single phase transformers A and B rated at 250KVA each are operated in parallel on both sides. Percentage impedance for A and B are (1+j6) and (1.2+j4.8) respectively. Compute load shared by each when the total load is 500KVA at 0.8 p.f. lagging. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Give six points of comparison between auto transformer and two winding transformer. |  |
| b) | Write a short note on high frequency transformer with respect to construction and characteristics. |  |
| c) | With a neat diagram explain the connection and operation of “OFF load tap changer”. |  |

***********