## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Materials & Processes(FE404) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is the difference between cast iron, wrought iron and steel? |  |
| b) | Explain the purpose of alloying steels. Also list down some common alloying elements and their effects. |  |
| c) | Discuss the need of heat treatment of steels. |  |
| d) | Explain the effects of absorption of various gases in welding. |  |
| e) | Briefly explain the L-D process of steel manufacture |  |
| f) | Explain refractory materials and ceramics. |  |
| g) | Differentiate between sand blasting and shot blasting. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat sketch explain the Iron-Carbon equilibrium diagram of steel upto 2% Carbon and explain the various phases in it. |  |
| b) | Discuss the effects of carbon, silicon, manganese, sulphur, and phosphorus on the properties of steel. |  |
| c) | What is High speed steel? Explain the effects of alloying elements on high speed steel. Give the application of 18-4-1 H.S.S. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the various metallurgical effects of welding? Also explain causes & prevention of weld cracking. |  |
| b) | Why is Tempering followed by Hardening? Explain the process of Tempering along with effects and types of Tempering. |  |
| c) | Give mechanical properties, description and applications of any three classes of cast iron. (2 points each) |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss the phenomenon of Age hardening in Al-Cu system with a neat sketch. |  |
| b) | Describe in brief Hot working processes. Explain any one Hot working process with the help of neat sketch. |  |
| c) | Explain the process of recovery, recrystallization and grain growth in the Annealing process. Compare full annealing with process annealing. |  |
| Q.No.5. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Discuss briefly grey cast iron & white cast iron. |  |
| b) | Briefly describe the Cold working processes. |  |
| c) | Write short note on Galvanising & Powder coating. |  |
| d) | Write short note on chemical cleaning of steel. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Corrosion of welds |  |
| b) | Prevention of gaseous contamination in welds |  |
| c) | Heat resisting steels |  |
| d) | Pig iron & its manufacture |  |
| e) | Insulating materials |  |

***********