## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Engineering & Technology

Subject: Engineering Mechanics(CS301) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define force. Explain how force can be resolved into force & couple. |  |
| b) | Explain Moment of couple with a neat sketch. |  |
| c) | Explain the various types of beams. |  |
| d) | Define fiction. State the laws of static friction. |  |
| e) | Explain the application of D’Alembert’s principle. |  |
| f) | State and explain the law of conservation of momentum. |  |
| g) | Explain Kinetic energy and Potential energy with their equations. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the resultant in magnitude and direction of the force system shown in fig.1. |  |
| b) | Find the resultant and equilibrant force of the following: 80N at 20 100N at 80 200N at 150 70N at 280 |  |
| c) | Determine the magnitude, direction and position of the parallel system of forces shown in fig.2. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find forces in the members AB, BC, CD and unknown weight ‘W’ of the system shown in fig.3. |  |
| b) | Find the reaction at the supports A & B for the beam loaded as shown in fig.4. |  |
| c) | Using method of joints, determine the forces in all the members of the truss shown in fig.5. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A force of 250N pulls a body of weight 300N up inclined plane, the force being applied parallel to the plane. If the inclination of the plane to the horizontal is 15, find the coefficient of friction. |  |
| b) | A ladder of 5m length is supported against a rough wall at an angle  with the floor. The coefficient of friction at wall and floor is 0.2. If the weight of the ladder is W and acts at the centre of the ladder, find the angle  with the floor when the ladder is at the point of slipping. |  |
| c) | A mine cage of mass 1.2 tonnes starts from rest and moves down with constant acceleration. It descends 352m in 10 seconds. Find the tension in the cable supporting the cage. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Two particles of masses ‘M1’ and ‘M2’ are connected by a light extensible string passing over a light smooth pulley at the end of a smooth horizontal table. ‘M2’ is lying on the table & ‘M1’ hanging vertically downwards. Determine the resulting acceleration & tension in the string. |  |
|  |  | …..2/- |
|  | -2- |  |
| b) | A train has a mass of 200 tonne. If the frictional resistance amounts to 6N per tone mass, what pull must the engine exert in order to increase the velocity on a level track from 30 Kmph to 60 Kmph in 1.5 minutes. |  |
| c) | A pile hammer of weight 3KN falls from a height of 3.5m on the top of a pile. Find the impulsive force of the blow if the hammer comes to rest in 1/100 second. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | An underground water sump 5mx6mx3m filled with water is to be completely emptied in 10 hours. The water is lifted through an average height of 25m to an overhead tank. The pump has 70% efficiency. Find the rated power of the pump. |  |
| b) | Determine the work done in stretching a spring through a length of 100mm. The stiffness of the spring is 5N/mm. |  |
| c) | A body of mass 5 Kgs drops freely from a height of 60m and penetrates into the ground by 1.5m. Find the average resistance of penetration. |  |