## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Food Technology

Subject: Food Engineering-I(FT304)/(4265) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any ten of the following Questions: | 10x1.5=15 |
| --- | --- | --- |
| a) | Define Thermal Conductivity and give its unit. What is its relation to thermal resistance? |  |
| b) | State Stefan Boltzmann Law and name the terms therein. |  |
| c) | Give an expression for overall heat transfer coefficient, naming all terms therein. |  |
| d) | Why heat exchangers for gases usually have extended surfaces? |  |
| e) | Differentiate between arithmetic mean temperature difference and logarithm mean temperature difference in a heat exchanger. |  |
| f) | What is meant by evaporative cooling? |  |
| g) | What is the RH of air having DBT of 25C and WBT of 20C? |  |
| h) | Name three advantages of vacuum concentration over atmospheric pressure concentration. |  |
| i) | Distinguish between free and bound moisture. |  |
| j) | Name the types of air-flow used in dehydration. |  |
| k) | Explain the meaning of equilibrium moisture content. |  |
| l) | State the principle of vapour compression refrigeration system. |  |
| m) | Define quick freezing and state two advantages of this process over slow freezing. |  |
| n) | State the advantages of steam distillation. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain how heat is transferred through a plane two-layered composite wall, and derive and expression for the same. Calculate the heat loss per square metre of the surface of a brick wall, 0.5m thick when the inner surface is 400K and outside surface is 310K. The thermal conductivity of brick is 0.7 W / mK. |  |
| b) | Compare tubular and plate heat exchangers with respect to characteristics and uses. |  |
| c) | With the help of a neat diagram explain the working of any steam heated tubular heat exchanger. What are the main advantages and drawbacks of this heat exchanger? |  |
| Q.No.3. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | List the factors that will improve conduction of heat to a liquid flowing in forced convection, and explain the same. |  |
| b) | Write a brief note on mechanically aided heat exchangers – their construction and application. |  |
| c) | Draw a neat diagram of a double pipe heat exchanger and explain what is counter current and parallel flow. |  |
| d) | Explain the use of humidity charts. |  |
| e) | Name any three humidity measuring instruments. Write the principle and construction of any one of them. |  |
| f) | With the help of a neat sketch, explain the working of a Calandria evaporator. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Give an account of the different concentration equipment available and their applications. When is multi-effect evaporation preferred? Name the equipment you would prefer for the concentration of (i) apple juice (ii) liquid milk (iii) sugar cane juice in sugar manufacture. |  |
| b) | Name two main the continuous dryers used for dehydration of liquid foods. List the characteristics of the equipment and the foods for which they are most suitable. Give examples to illustrate your answer. |  |
| c) | Explain the construction, working and applications of a tray dryer. Why is this equipment found to be most flexible in its usage? Explain, giving suitable examples. |  |
| Q.No.5. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | Briefly explain the principle, working and applications of a fluidized bed dryer. |  |
| b) | List and explain different ways to increase the efficiency of tubular evaporators. Give examples to illustrate your answer. |  |
| c) | Explain, with the help of a graph, the principle of freeze drying. Briefly outline the operation of a freezer dryer. |  |
| d) | Explain the principle of absorption refrigeration and its application in industry. |  |
| e) | Draw a diagram and briefly explain the components and working of a vapour compression refrigeration cycle. |  |
| f) | Write a technical note on common refrigerants, their classification and selection. |  |
| Q.No.6. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | Explain the construction, working and uses of a plate freezer. |  |
| b) | Compare the construction and uses of a fluidized bed freezer and a blast freezer. |  |
| c) | List the ways frozen foods deteriorate on storage. Explain ways to minimize these losses. |  |
| d) | Write an explanatory note on cryogenic freezing of foods. |  |
| e) | Define distillation. Give a brief account of different types of distillation processes and the principle involved in each. (any three) |  |
| f) | Explain the process of fractionation, with the help of a diagram. Clearly label different areas of the fractionating column. State the uses of this process. |  |

***********