## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Electronics Engg./EC/MED/EI

Subject: Basic Electronics Engineering(CS308) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain characteristics of Zener diode when it is reverse biased. |  |
| b) | What is the function of capacitor filter? Define ripple factor. |  |
| c) | What do you mean by Alpha & Beta of a transistor? What is the relation them? |  |
| d) | What are the two biasing conditions of JFET? |  |
| e) | Draw the structure and characteristics of Depletion type MOSFET. |  |
| f) | Define propagation delay and noise immunity of Logic Families. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the circuit of capacitor filter with the help of waveforms. |  |
| b) | Explain the working of Zener diode voltage regulator circuit when load is constant but input voltage changes and when Input is constant but load changes. |  |
| c) | Define voltage regulation. Calculate % regulation if no load voltage is 12V and full load voltage is 11.9V. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the construction of NPN transistor and write the function of Emitter, Base and Collector region. What is the relation between these three currents? |  |
| b) | Draw CE configuration of NPN transistor and explain Input and Output characteristics. |  |
| c) | The dc current gain of a transistor in CE configuration is 100. Find its dc current gain in CB configuration. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the diagram of JFET Common Source configuration and explain the pinch-off region characteristics of JFET. |  |
| b) | Explain the parameters drain resistance, transconductance and amplification factor of JFET. |  |
| c) | Explain the operation of Enhancement type of MOSFET with the help of diagrams and characteristics. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the logic of CMOS NOR gate and explain the working with the help of truth table. |  |
| b) | Draw the diagram of EEPROM cell and explain how bit is stored and erased. |  |
| c) | Explain the circuit of MOS static RAM Cell. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain two important specifications of Zener Diode. |  |
| b) | Give 4 points of comparison between CE, CB and CC configuration. |  |
| c) | Explain current and voltage ratings of transistor. |  |
| d) | Give 4 points of comparison between BJT and JFET.. |  |

***********