## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Electrical Engg.

Subject: Illumination Engineering(EE612)/(5106) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State any six qualities of good lighting. |  |
| b) | What is meant by stroboscopic effect in artificial lighting? State how this effect is eliminated in fluorescent tube lighting. |  |
| c) | Define i) Luminous Intensity ii) M.H.C.P iii) M.S.C.P. |  |
| d) | Give three points of distinction between luminance and illuminance. |  |
| e) | Define the following: i) Direct Ratio ii) Uniformity Ratio iii) L.O.R |  |
| f) | State the requirements of good street lighting. |  |
| g) | Classify the floodlight projectors. Also state the basis for their classification. |  |
| h) | Give any three important features of sports lighting. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State and explain different types of lighting schemes. Also state two applications of each of these schemes. |  |
| b) | With the help of neat diagram, explain difference between i) Direct lighting system & Indirect lighting system. ii) Semi Direct lighting system & semi-Indirect lighting system. |  |
| c) | State and explain different types of reflections we come across in Illumination Engineering. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a)i) | State the different factors affecting selection of coefficient of utilization. (six points) | (3) |
| ii) | State the factors affecting selection of luminaires used in Interior lighting. | (3) |
| b) | State all the special features of entrance lighting, staircase lighting and corridor lighting. |  |
| c) | A hall 30m long and 12m wide is to be illuminated and illumination required is 50m candles. Five types of lamps having lumen outputs as given below are available. Watts; 100; 200; 300; 500; 1000. Lumens; 1615; 3650; 4700; 9950; 21500. Taking a depreciation factor of 1.3 and coefficient of utilization = 0.5, calculate the number of lamps needed in each case to produce required illumination. Out of above five types of lamps select most suitable type and design a suitable lighting scheme and make a sketch showing location of lamps. Assume a suitable mounting height and calculate space height ratio of lamps. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a)i) | Two lamps are hung at a height of 9m from the floor level. The distance between the lamps is 2m. Lamp1 is of 500cp. If the illumination on the floor vertically below this lamp is 20 lux, find the candle power of lamp 2. | (4) |
| ii) | A 250V lamp has a total flux of 3000 lumens and takes a current of 0.8A. Calculate i) lumens per Watt ii) MSCP per Watt. | (2) |
| b) | For a fluorescent tube mounted at a height of ‘H’ metres from the working plane and having luminous Intensity of Io candles per metre length of the tube and for an aspect angle  specified in radians, state the expression for i) The illumination level at a point on working plane which is exactly below one of the edges of a tube. ii) Illumination level at a point on working plane which is exactly below centre of tube. iii) Illumination level at a point on the working plane which is located far away from one end of tube. Draw necessary diagrams. |  |
| c) | State and explain two laws of illumination. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the different types of streetlight luminaires. Also state the basis of their classification & one application of each type of luminaire. Also arrange these luminaires in ascending order of their presence of glare. |  |
| b) | Explain the steps in street lighting design by using any one method. |  |
| c)i) | Explain how street lighting is different from indoor lighting? | (3) |
| ii) | What are the requirements of vehicle driver? | (3) |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State and explain all the steps required in flood lighting design. |  |
| b) | With reference to flood lighting: i) State the types of lamps used ii) State the different types of locations of flood light projectors. |  |
| c)i) | Give three important features of Auditorium lighting. | (3) |
| ii) | Give three important features of Hospital lighting. | (3) |

***********