## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Mechanical Engg.

Subject: Fluid Machinery(4040) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | A jet of water 15mm in diameter issues with a velocity of 20m/s and impinges normally on a stationery flat plate. Find the force exerted by the jet on the plate. |  |
| b) | Define overall efficiency of a turbine. |  |
| c) | What do you understand by net positive suction head of a centrifugal pump? |  |
| d) | State the function of Air vessels in reciprocating pump. |  |
| e) | State applications of gear pump. |  |
| f) | Name two types of oil seals and state their uses. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a schematic diagram of a Francis turbine and explain its construction and working. |  |
| b) | State the advantages and disadvantages of a Kaplan Turbine over a Francis Turbine. |  |
| c) | What is cavitation? How it can be avoided in a reaction turbine. Explain any method to avoid cavitation in detail. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Which points should be considered while selecting right type of hydraulic turbine for hydroelectric power plant? |  |
| b) | Explain the working of a single stage centrifugal pump with a neat sketch. |  |
| c) | Explain the significance of characteristics curve of centrifugal pump. Draw operating characteristics curve of a centrifugal pump. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is priming? Why is it necessary? How are small and large centrifugal pumps primed? |  |
| b) | Define indicator diagram. Draw an indicator diagram considering the effect of friction in suction and delivery pipes. |  |
| c) | Describe the principle and working of a single acting reciprocating pump. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the construction and working of a rotary vane pump. |  |
| b) | Describe a quick exhaust valve, with the help of a neat sketch. |  |
| c) | With the help of a neat sketch, explain regenerative type of a hydraulic circuit. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | How is a sequence valve used in a hydraulic circuit? Explain a sequence valve with help of a neat sketch. |  |
| b) | What is the importance of a flow control valve in a pneumatic circuit? Explain the different types of actuation systems of flow control valves. |  |
| c) | Define slip, percentage slip and negative slip of a reciprocating pump. |  |

***********