## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Electrical & Electronics Engg.

Subject: Fundamentals of Refrigeration & Airconditioning(5090) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State and explain Kelvin-Planck statement of second law of thermodynamics. |  |
| b) | Write the designation of C2Cl3F3. |  |
| c) | Define following Psychrometry terms: i) Relative humidity ii) Dew point temperature |  |
| d) | State and explain function of Limit switch. |  |
| e) | Define Coefficient Of Performance of Refrigeration system. |  |
| f) | Draw Pressure-Enthalpy ‘P-H’ diagram, when the vapour is superheated after compression. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State and explain desirable properties of an ideal refrigerant. |  |
| b) | With help of a neat sketch explain vapour compression system. |  |
| c) | Explain the effect of following factors on performance of vapour compression system: i) Effect of suction pressure ii) Effect of sub-cooling of liquid |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain construction and working of an Electrolux refrigerator system with help of a neat sketch. |  |
| b) | State the function of a condenser in a refrigeration system. Explain with a neat sketch water cooled condenser. |  |
| c) | Discuss the construction and operation of a thermostatic expansion valve with help of a neat sketch. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With help of a neat sketch explain construction and working of a window air conditioner. |  |
| b) | Describe with a neat sketch a cold storage plant. |  |
| c) | Explain the working of an air washer used in air conditioning system with help of a neat sketch. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define following terms used in psychrometrics and explain its significance: i) Wet bulb depression ii) Specific humidity. |  |
| b) | With the help of psychrometric chart explain the following process: i) Mixing ii) Cooling and humidification. |  |
| c) | Define Room Sensible Heat Factor. With help of a neat sketch explain its significance and use. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain construction and working of a Domestic refrigerator. |  |
| b) | What is the function of a filter in an air conditioning system? How are they classified? Explain any one with help of a neat sketch. |  |
| c) | State the functions of the following components in an absorption system: i) Absorber ii) Rectifier iii) Analyser. |  |

***********