## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Mechanical Engg./A

Subject: Hydraulic Machinery(ME404)/(4053) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define i) Specific weight ii) Specific gravity iii) Capillarity. |  |
| b) | Define i) Atmospheric pressure ii) Guage pressure iii) Absolute pressure. |  |
| c) | What is total pressure and centre of pressure on an immersed surface? |  |
| d) | Define i) Steady flow ii) Uniform flow iii) Turbulent flow. |  |
| e) | What is priming of centrifugal pumps? State any two methods of priming. |  |
| f) | State any three functions of air vessels in reciprocating pumps. |  |
| g) | Define Reynolds number. State its significance. (only for students of revised scheme) |  |
| h) | Differentiate between Impulse and Reaction Turbines. Give one examples of each. (only for students of revised scheme) |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | An isosceles triangular plate of base 3m and altitude 3m is immersed vertically in an oil of specific gravity 0.8. The base of the plate coincides with the free surface of oil. Determine i) Total pressure on the plate ii) Position of centre of pressure. |  |
| b) | Water is flowing through a pipe having diameters 600mm and 400mm at the bottom and upper end respectively. The intensity of pressure at the bottom end is 350KN/m2 and the pressure at the upper end is 100KN/m2. Determine the difference in datum head if the rate of flow through the pipe is 60 litres/second. |  |
| c) | A horizontal pipe 150mm in diameter is joined by sudden enlargement to a 225mm diameter pipe. Water is flowing through it at the rate of 0.05m3/s. Find i) Loss of head due to abrupt expansion ii) Pressure difference in the two pipes. (only for students of revised scheme) |  |
| c) | A rectangular notch 2m wide has a constant head of 500mm. Find the discharge over the notch, if coefficient of discharge is 0.62. (only for students of old scheme) |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A single acting reciprocating pump operating at 120 r.p.m. has a piston diameter of 200mm and a stroke of 300mm. The suction and delivery heads are 4m and 20m respectively. If the efficiency of both suction and delivery stokes is 75 percent, determine the power required by the pump. |  |
|  | only for students of revised scheme: |  |
| b) | With the help of a neat labelled sketch, explain the construction and working of a Pelton wheel turbine. |  |
|  | -2- |  |
| c) | With the help of a neat sketch, explain the governing mechanism in Francis turbine. |  |
|  | only for students of old scheme: |  |
| b) | With the help of a neat sketch, explain the construction & working of Hydraulic Intensifier. |  |
| c) | Explain construction & working of a gear pump with help of a neat sketch. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The following are the results of tests conducted on a centrifugal pump. Negative pressure head in suction pipe close to pump = 250mm of mecury; Pressure intensity in delivery pipe close to pump = 0.14N/mm2; Difference of level between the two gauges = 0.45 metre; Shaft power of the electric motor = 22KW; Rate of discharge = 0.085 m3/s; Diameter of suction pipe = 200 mm; Diameter of delivery pipe = 150mm. Calculate the overall efficiency of the pump. |  |
| b) | State the different faults that occur in centrifugal pumps. Explain the cause of each fault and state their remedies. |  |
| c) | With the help of a neat sketch, explain the construction and working of a rotary vane pump. State its applications. |  |
| Q.No.5. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | A differential manometer connected at the two points A and B in a pipe containing an oil of specific gravity 0.9, shows a difference in mercury levels of 150mm. Find the difference in pressure at the two points A and B. |  |
| b) | A circular plate of diameter 1.2m placed vertically in water in such a way that the centre of the plate is 2.5m below the free surface of water. Determine the position of centre of pressure. |  |
| c) | Explain the terms dynamic viscosity and kinematic viscosity. State their units. |  |
| d) | Explain hydraulic gradient line and total pressure line. |  |
| e) | State Pascal’s law. Explain its application with a suitable example. (only for students of revised scheme) |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Define i) Coefficient of velocity ii) Coefficient of contraction iii) Coefficient of discharge iv) Vena Contracta. |  |
| b) | Define water hammer. What are its causes, effects and remedies? |  |
| c) | Explain the meaning of Net Positive Suction Head. |  |
| d) | Draw a neat sketch of reciprocating pump and explain its working. |  |
| e) | With the help of a neat sketch, explain the working of hydraulic accumulator. |  |

***********