## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Common

Subject: Elements of Mechanical Engineering(3004) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State various materials used in making belts used in drives. |  |
| b) | What are the functions of cooling system in IC engine? |  |
| c) | Explain the functions of blow-off cock. |  |
| d) | State the uses of compressed air. |  |
| e) | Why is it necessary to have lubrication system? |  |
| f) | State & explain Bernoulli’s theorem. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State advantages and disadvantages of a chain drive. |  |
| b) | Classify rolling contact bearings. |  |
| c) | With help of a neat sketch explain working of a compound gear train. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Differentiate between petrol and diesel engine. |  |
| b) | Discuss any fuel system employed in an I.C. engine. |  |
| c) | With help of a neat sketch explain working of an economiser in a boiler. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe with a neat sketch, the working of an air-motor. |  |
| b) | Classify air compressors. Describe the working of a single stage reciprocating air compressor. |  |
| c) | State the desirable properties of a good lubricant. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What do you understand by the term cavitation in pump? What are the reasons for cavitation? How can cavitation be avoided? |  |
| b) | With help of a neat sketch discuss the loss of head in a pipe due to contraction. |  |
| c) | State the difference between single stage and multi-stage pumps. Describe a multi-stage pump with impellers in series. |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Fusible plug |  |
| b) | Methods of lubrication |  |
| c) | Journal thrust bearing |  |
| d) | Ignition system of petrol engine |  |

***********