## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Mining Engg.

Subject: Mine Ventilation(MN503)/(4297) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Give the permissible concentration of all the underground mine gases in air. |  |
| b) | Differentiate between homotropal & anti-tropal ventilation. |  |
| c) | What is turbulent air flow? Give reasons for its occurrence in underground mine? |  |
| d) | Give the advantages of splitting. |  |
| e) | How is reversal of air done in centrifugal and axial flow fans? |  |
| f) | Mention the purposes of booster fan in underground mines. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the physical and chemical properties of methane? Explain the coward’s diagram. |  |
| b) | Explain the principle & construction of flame safety lamp? |  |
| c) | i) Give the standards of ventilation. ii) What is the effect of humidity on underground mine worker? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the term motive column. A mine is worked by an intake shaft of 300m deep with a temperature of 20°C and a return shaft of 150m deep having 26°C. Calculate the NVP assuming the average barometric pressure to be 750mm of w.g. |  |
| b) | Explain the construction of air crossing with a neat sketch. |  |
| c) | i) Explain the operation of pitot tube. ii) What is the equivalent resistance of two galleries of same resistance ‘R’ connected in parallel? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the principle & construction of centrifugal fan with a neat sketch. |  |
| b) | A 4m diameter centrifugal fan with backward blades delivers 6000m3/min of air when running at 300rpm. If the blade angle is 35° and width of fan at periphery is 1.2m, calculate the theoretical w.g. developed by the fan. Assume air density to be 1.2 Kg/m3. |  |
| c) | i) Give advantages of axial flow fan over centrifugal fans. ii) A mine is ventilated by a fan running at 300 rpm which produces 6000m3/min at 70 mm w.g. What would be the new quantity & pressure when the fan speed is increased to 400 rpm? |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A total quantity of 200m3/min of air is passing through two splits. One airway is 3m x 2.5m and 100m long & the other with similar lining is 3.5m x 3m and 125m long. Calculate the quantity of air passing in each split? | …….2/- |
|  | -2- |  |
| b) | With a neat sketch explain the reversible method of auxiliary ventilation in a long heading. |  |
| c) | Mention the duties of ventilation officer. What precautions are taken during the ventilation survey? |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | What are the different sources of heat in underground mines? |  |
| b) | Give the sources of air leakage in underground mines. How do you prevent it? |  |
| c) | Explain the design consideration in construction of an Evasee. |  |
| d) | Explain the characteristic curves of an axial flow fan. |  |

***********