## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Electrical Engg.

Subject: Distribution & Estimation(EE511)/(4108) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Compare DC 2 wire system with single phase 2 wire AC system. |  |
| b) | State the factors to be taken into consideration while selecting size of distributor. |  |
| c) | Draw a neat sketch of Guy assembly & label its parts. |  |
| d) | Define wiring system. List out material required for carrying out cleat wiring & state its two disadvantages. |  |
| e) | State the factors affecting earth resistance. |  |
| f) | State the value of permissible voltage drop in various part of distribution system for urban, suburban & rural area. |  |
| g) | List out types and size of conductor used for Rural electrification as per REC |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Compare radial, ring main & interconnected system on the basis of load, reliability of supply & economy. |  |
| b) | i) Explain how ac distribution calculations differ from those of dc distribution. ii) A single phase ac distribution line has two parallel conductors 3m apart, the radius of each conductor being 1cm. Calculate the loop inductance per Km length of the line, if the material of the conductor is copper. | (3) (3) |
| c) | A single phase ac distributor 2Km long supplies a load of 120A at 0.8pf lagging at its for end and a load of 80A at 0.9pf lagging at its mid point. Both power factors are referred to the voltage at the far end. The resistance and reactance per Km (go and return) are 0.05Ω & 0.1Ω respectively. If the voltage at far end is maintained at 230 Volts, calculate i) Voltage at the sending end ii) Phase angle between voltage at the two ends. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the method of estimation of capacity of distribution transformer for a particular locality. |  |
| b) | Write a short note on guarding of overhead lines. Draw sketches showing i) Cradle & Cage type guarding ii) Guarding at road crossing iii) Guarding for railway lines. |  |
| c) | Estimate the quantity of material required for erecting 1Km of overhead 11KV, 50Hz line using steel poles of 11m height and ACSR conductor of 6/1 x 2.59mm with an average span of 120m. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain SWER system of rural electrification. List out its advantages and disadvantages. |  |
| b) | Explain how voltage drop is calculated in radial distributor using voltage regulation constant. |  |
| c) | Explain the method of installing concealed conduct wiring system. State its advantage & disadvantages. Give its area of applications. | …..2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the method of i) determination of light point and fan point & socket outlet points ii) number of sub circuit iii) rating of main switch & distribution board iv) size of cable, for domestic wiring |  |
| b) | Explain buy way system in industrial wiring installations and state its advantages and disadvantages. |  |
| c) | Two ac 3 415V 50Hz squirrel cage induction motor rated 15HP & 10HP are to be installed in a workshop. Determine i) size of the cable required ii) rating of the main switch iii) rating of the motor switches required. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain ‘Cable system’ wiring in industrial installation. State its advantages and disadvantages. |  |
| b) | With the help of neat diagram explain pipe earthing. |  |
| c) | i) What is double earthing? What are its advantages? ii) Define earthing. Explain the purpose of earthing. | (3) (3) |

***********