## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Welding Technology-II(FE403)/(4240) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List the base metals welded by Gas Tungsten Arc Welding (GTAW). |  |
| b) | Compare Argon and Helium as shielding gases. |  |
| c) | Explain in brief the concept of self adjusting arc in Gas Metal Arc Welding. |  |
| d) | What are the effects of operating variables in operating Flux Cored Arc Welding(FCAW)? |  |
| e) | State the applications of Atomic hydrogen welding. |  |
| f) | Define weldability. What is carbon equivalent? |  |
| g) | List the factors influencing procedure of maintenance welding. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Sketch at least four weld joint design for Gas Tungsten Arc welding (GTAW). |  |
| b) | Explain the manual Gas Tungsten Arc Welding (GTAW) techniques. |  |
| c) | With a neat sketch explain the welding torch used in Gas Metal Arc welding (GMAW). |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the principle of operation of Submerged Arc Welding (SAW). |  |
| b) | Explain at least two types of weld backing used in Submerged Arc Welding (SAW). |  |
| c) | What are the advantages and disadvantages of Electro Slag welding? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain operating procedure of Electro slag welding. |  |
| b) | With a neat sketch explain the principle of operation of ultrasonic welding. |  |
| c) | Explain the principle of operation of Resistance seam welding with a neat sketch. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the advantages and limitations of Resistance spot welding. |  |
| b) | Explain the causes of weld distortion. |  |
| c) | Explain at least two methods used for control of weld distortion. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Importance of Carbon Equivalent |  |
| b) | Welding Jigs & Fixtures |  |
| c) | Quick acting clamps |  |
| d) | Rebuilding of worn out shafts |  |

***********