## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Fabrication Process-I(FE303)/(4233) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the principle of micrometer. |  |
| b) | What are possible errors while using scriber? |  |
| c) | Draw a neat sketch of vernier depth gauge and label its parts. |  |
| d) | Explain the use of dividers and compasses in marking. |  |
| e) | Differentiate between radius and feeler gauge. |  |
| f) | List three important safety precautions to be observed during use of Power Hacksaw machine. |  |
| g) | State the procedure for extinguishing flame in flame cutting process. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the following errors and how do we take care to avoid them: i) Loading error ii) Parallax error iii) Instrumental errors. |  |
| b) | i) If main scale of a vernier caliper has 49 divisions and vernier scale has 50 divisions with first and last division on both scales coinciding, what is least count? ii) Give main scale reading and Vernier scale coincident division number to show 28.12. |  |
| c) | Sketch the following and explain their uses: i) Dividers and compass ii) Angle plate iii) Magnetic Vee block. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe a universal bevel protector with a neat sketch. |  |
| b) | Explain the construction and use of the following: i) Plumb line ii) Surveyor’s level iii) Water line. |  |
| c) | Explain the use of data for marking out pipe flanges. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain Roundness Testing using fixture and dial indicator method. |  |
| b) | With the help of a neat sketch explain the use of pipe square. |  |
| c) | What is the need for templates? What are box templates? |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain construction and working of “Reciprocating Power Hacksaw”. |  |
| b) | Compare flame cutting with shearing. |  |
| c) | Discuss any three commonly used gases in flame cutting. |  |
| Q.No.6. | Write short note on any three: | 3 x 4 = 12 |
| a) | Abrasive cutting machine |  |
| b) | Use of beam trammels |  |
| c) | Contour gauges |  |
| d) | Steel templates |  |
| e) | Ways of expressing Tolerance |  |

***********