## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Shipbuilding Engg.

Subject: Basic Ship Theory-I(SH301)/(4331) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short notes on any three: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Hovercraft |  |
| b) | Rise of floor and flat of keel |  |
| c) | Underwater volume |  |
| d) | Half breadth plan |  |
| e) | Free surface effect |  |
| f) | Tonnes per cm immersion |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write the salient features of i) Container ship and ii) Fishing trawler. |  |
| b) | A ship sails with displacement 3420 tonnes and KG = 3.75m. During the voyage bunkers were consumed as follows: 66 tonnes (KG = 0.45m) and 64 tonnes (KG = 2m) Find the KG at the end of the voyage. |  |
| c) | A ship’s length at the waterline is 120m when floating on an even keel at a draft of 4.5m. The maximum beam is 20m. If the ship’s block co-efficient is 0.75, find the displacement in tonnes at this draft in salt water. |  |
| Q.No.3. | The areas of a ship’s water planes, commencing from the load draft of 24 metres, and taken at equal distances apart are: 2000, 1950, 1800, 1400, 800, 400 and 100 sq.m. respectively. The lower area is that of the ship’s outer bottom. Find the displacement in salt water and the height of the centre of buoyancy above the keel. | (12) |
| Q.No.4. | Answer the following Questions: | 2 x 6 = 12 |
| a) | A ship has an initial GM=0.5m. When a mass of 25 tonnes is shifted transversely a distance of 10m across the deck, it causes a deflection of 0.4m in a 4m plumb line. Find the ship’s displacement. (Only for students of revised scheme) |  |
| a) | Explain the terms: i) TRIM ii) Maximum GZ. (Only for students of old scheme) |  |
| b) | The second moment of a ship’s water-plane area about the centre line is 20,000 m4 units. The displacement is 7000 tonnes whilst floating in dock water of density 1008 kg per cu.m. KB = 1.9m and KG = 3.2m. Calculate the initial meta centric height. |  |
| Q.No.5. | Answer any one of the following Questions: | (12) |
| a) | A ship of 12,000 tonnes displacement is 150m long at the waterline. The half ordinates of the water plane at equidistant intervals from forward: 0, 4, 8.5, 11.6, 12.2, 12.5, 11.6, 5.2, 2.4 and 0.3 m respectively. Calculate the transverse BM. |  |
| b) | A ship of 8153 tonnes displacement has KM=8m, KG=7.5m and has a double bottom tank 15m x 10m x 2m which is full of salt water ballast. Find the new GM if this tank is now pumped out till half empty. | …….2/- |
|  | -2- |  |
| Q.No.6. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Explain the following terms: - Hydrostatic curves - L C F |  |
| b) | The waterplane areas of a ship at 1.25m intervals of draught commencing from 7.5m waterline are: Draught; 7.5; 8.75; 10.00; 11.25; 12.50. Water plane area (m2); 1845; 1690; 1535; 1355; 1200. |  |
|  | Determine the mass to be added to increase the draft from 10.00m to 10.20m. |  |

***********