## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Civil Engg./CT

Subject: Irrigation Engg.(CE601)/(4017) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1 | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define lift, perennial and inundation irrigation. |  |
| b) | Explain hydrological cycle with neat sketch. |  |
| c) | What do you mean by Duty, Delta and Base period of a Crop? |  |
| d) | Which are the forces acting on gravity dam? |  |
| e) | Distinguish between hydraulic filled dam and rolled filled dam. |  |
| f) | Which are the losses in storage reservoir? |  |
| g) | Distinguish between aqueduct super passage. |  |
| h) | Write down the advantages and disadvantages of minor irrigation schemes. |  |
| Q.No.2 | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the aid of neat labelled sketch, explain the measurement of rainfall by weighing bucket type automatic rain guage. |  |
| b) | Derive the relationship between Duty, Delta and Base period. Write any two factors affecting duty. |  |
| c) | Define runoff. Which are the factors affecting runoff? Write down the Inglis formula for ghat and non-ghat areas. |  |
| Q.No.3 | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Distinguish between: i) Gross-command area and culturable command area ii) Time factor and capacity factor |  |
| b) | Which are the different types of failure in gravity dam? |  |
| c) | Determine the head discharge of a canal from following data. The value of time factor may be taken as 0.75. Crop; Base period in day; Area in hectares; Duty in hect/cumec. Rice; 120; 4200; 1600. Wheat; 120; 3700; 2100. Sugarcane; 120; 3200; 1300. |  |
| Q.No.4 | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Fix the control levels of a reservoir using the following Data: Culturable command area: 160 Ha.m. Gross duty: 230 Ha/Mm3 Dead storage at L.S.L.: 20Ha.m. Tank losses: 12% of crop water requirement Free board: 2.0m |  |
| Contour RL(m); 15; 30; 60; 90; 120; 150; 180; 210; 240; 270; 300; 330; 360; 390; 420. Capacity (Ha.m.); 10; 18; 33; 52; 65; 78; 85; 98; 106; 118; 132; 152; 165; 175; 190. |
| b) | Explain sedimentation in reservoir. Which are the preventive measures to control sedimentation? |  |
| c) | With the aid of neat sketches (section of canal), explain the canal in cutting, embankment, partial cutting and partial embankment. |  |
| Q.No.5 | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Which are the different types of failure in earthen dams? |  |
| b) | What are the preliminary surveys and final surveys conducted before selecting Dam site? | …..2/- |
|  | -2- |  |
| c) | Which are the different types of lining? Write any two advantages and disadvantages of coral lining. |  |
| Q.No.6 | Write short notes on any three: | 3 x 4 = 12 |
| a) | Direct and storage reservoir |  |
| b) | Energy dissipaters |  |
| c) | Drain crossing over a canal |  |
| d) | Remedial measures for strengthening dam |  |

***********