## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Electrical Engg./E&EE

Subject: D.C. Machines(4109) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define (i) Pole Pitch (ii) Front pitch (iii) Back pitch. |  |
| b) | Give three points of distinction between lap and wave winding. |  |
| c) | Classify the various types of losses taking place in a d.c. generator. |  |
| d) | Draw a neat diagram of third brush generator and state its applications. |  |
| e) | What is the need for parallel operation of d.c. generators? |  |
| f) | Explain the importance of back emf in a d.c. motor. |  |
| g) | Classify the various losses in DC motor. Which of these losses are constant losses and variable losses? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List out different parts of D.C. machine and explain function of each part in detail. |  |
| b) | Write a note on DC welding generator. |  |
| c) | Two d.c. shunt generators run in parallel to supply together 2,500A. The machines have armature resistance of 0.04  and 0.025, field resistance of 25 and 20 and induced emfs of 440V and 420V respectively. Find the busbar voltage and output of each machine. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the classification of D.C. Generator with respect to excitation system. |  |
| b) | Explain the process of voltage build up in a DC shunt generator. |  |
| c) | A 4-pole dc shunt generator with lap connected armature supplies a load of 100A at 200V. The armature resistance is 0.1  and shunt field resistance is 80. Find (i) Total armature current (ii) Emf generated. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat diagrams explain the classification of D.C. motors based on connection of field and armature windings and write the voltage and power equations for each of them. |  |
| b) | With a neat sketch, explain the Ward-Leonard method of speed control of DC shunt motor. Give its advantages and disadvantages. |  |
| c) | With a neat diagram explain the construction and working of 4 point starter. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write the expression for efficiency of DC motor and derive the condition for maximum efficiency. Also define different efficiencies of motor. |  |
|  |  | …….2/- |
|  | -2- |  |
| b) | Write short notes on the following topics: - Necessity of starter for starting dc motor - Principle of operation of dc motor |  |
| c) | A 4-pole, 240V, wave connected shunt motor gives 1119 KW when running at 1000 rpm and drawing armature and field currents of 50A and 1A respectively. It has 540 conductors. Its resistance is 0.1 . Assuming a drop of 1 volt per brush, find (i) total torque (ii) useful torque (iii) rotational losses and (iv) efficiency. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram explain the principle of commutation and reasons for poor commutation. Briefly explain any one method for improving commutation. |  |
| b) | What is armature reaction? Explain its effect on performance of DC machine. |  |
| c) | A d.c. shunt generator delivers 195A at a terminal voltage of 250V. The armature and shunt field resistance are 0.02 and 50 respectively. The iron & frictional losses equal 950W. Find (i) Generated emf (ii) Copper losses (iii) Prime mover output (iv) Commercial efficiency. |  |

***********