## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Mining Engg.

Subject: Underground Mining Machinery(MN403)/(4294) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Write composition of steel wire rope. |  |
| b) | Sketch and label elements of a winding system. |  |
| c) | Enumerate suitability conditions of shearer. |  |
| d) | State essential requirement of a good haulage track. |  |
| e) | Enumerate advantages of mechanisation. |  |
| f) | List machines used in underground coal mines. (only for students of revised scheme) |  |
| f) | Explain coal mine Regulation 71. (only for students of old scheme) |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With sketch explain construction of Shearer. |  |
| b) | Explain construction of LHD. |  |
| c) | i) Enumerate suitability condition of coal cutting machine. ii) Explain rope lay. |  |
| Q.No.3. | Answer the following Questions: | 2 x 6 = 12 |
| a) | With sketch explain direct rope haulage. |  |
| b) | The data given below are available in respect to direct haulage at the top of incline. Length of incline – 800m; Gradient – 1 in 7; Number of loaded tubs per set –8; Weight of load - 1tonne; Weight of empty tub- 0.5 tonne; Speed of rope - 10Km/hr; Weight of rope - 1.5Kg/m; Coefficient of tube friction - 1/50; Coefficient of rope friction - 1/10. Calculate the maximum size of electric motor in KW required to work the haulage. Assume gear efficiency 75% and motor efficiency is 90%. |  |
|  | ### OR |  |
| c) | Calculate the carrying capacity of a 900mm belt conveyor, when cross-section of load is 0.08 square metre and belt moves at a speed of 8Km/hr. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain safety device in rope haulage system. |  |
| b) | With sketch explain king safety hook. |  |
| c) | Compare drum winding with friction winding. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With sketch explain tower mounted Koepe winding system. |  |
| b) | Explain purposes of headgear and material used in construction. |  |
| c) | Explain advantages and disadvantages of locomotive haulage. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.6. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Find the diameter of rope to be used in winding system given below. Weight of skip – 2 tonnes; Weight of mineral in skip – 2.5 tonnes; Depth of the shaft – 500m; Ultimate tensile strength of rope - 160Kg/mm2; Factor of safety – 8; Rope - locked coil rope. |  |
| b) | Explain role of maintenance in mining. ((only for students of revised scheme) |  |
| b) | Explain roof bolting machine. (only for students of old scheme) |  |

***********