## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Mechanical Engg.

Subject: Automobile Engineering(ME614) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the classification of automobiles based on i) Use ii) Capacity (iii) Body style. |  |
| b) | State 3 advantages of monocoque design over conventional/framed chassis. |  |
| c) | State 3 different between multipoint injection system and throttle body injection system. |  |
| d) | What is torque converter? How does it differ from gearbox? |  |
| e) | Explain 20-hour rating of a battery. |  |
| f) | State 3 important reasons for car air conditioner not producing sufficient amount of cooling. |  |
| g) | Describe the differences between drum and disc brakes. |  |
| h) | Name different types of steering gearboxes. Draw sketch of any one type. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State different types of Chassis frames. Explain any one type giving a neat sketch. |  |
| b) | Explain the procedure of setting spark ignition timing of petrol engines. |  |
| c) | Name the different sensors used in electronic fuel injection management system. Explain the working of any two sensors. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction and working of automatic transmission. |  |
| b) | Explain the construction of a transfer case. |  |
| c) | Explain the construction and working of a fluid coupting. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction and working of any one type of starting system of an automobile. |  |
| b) | Explain the construction of any one type of compressor used in automobile car air conditioner. |  |
| c) | Explain the construction and working of Pneumatic brake system. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction of steering linkage for rigid axle suspension system. |  |
| b) | Explain the construction of Double Wishbone type independent suspension system. |  |
| c) | Explain the construction and working of a hydraulic hoist. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Write a brief report on history of automobiles. |  |
| b) | Draw a wiring diagram of lighting and electrical instrument system of an automobile. | …..2/- |
|  | -2- |  |
| c) | State the desirable properties of a hydraulic brake fluid. |  |
| d) | Define i) Caster ii) Camber iii) King pin inclination iv) Toe in. |  |
| e) | Describe the construction of leaf springs. |  |
| f) | State the application of piston ring compressor. State the various types of spanners used in automobile workshop. |  |

***********