## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Civil Engg./CT

Subject: Transportation Engineering-I(4006) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the various types of Transportation systems. |  |
| b) | What precautions are to be taken while rolling earth roads? |  |
| c) | What are the surface dressings adopted for construction of bituminous roads? |  |
| d) | State the advantages of concrete roads over Asphalt roads. |  |
| e) | State the factors affecting the selection of site for a bridge. |  |
| f) | With neat sketches state the different types of tunnel shapes. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain classification of roads based on materials used in construction. |  |
| b) | What is super elevation? Why is it necessary to be provided for roads? |  |
| c) | What are the various defects in WBM roads? How are the WBM roads maintained? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction procedure of a Bituminous concrete road. |  |
| b) | Explain: i) Prime coat ii) Seal coat iii) Tack coat. |  |
| c) | Name the various types of joints provided in cement concrete roads. Why is it necessary to provide these joints? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat cross section of a hill road and mark all its component parts. Also write a note on floating gradient. |  |
| b) | Explain with neat sketches how surface and subsurface drainage of roads is done. |  |
| c) | Explain “Free board” provided for bridges. State its values for different types of bridges. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Why are Bearings provided in Bridges? With neat sketches explain any two types of bearings? |  |
| b) | i) Differentiate between a causeway and a culvert. ii) Draw a neat sketch of a box culvert and mark its component parts. |  |
| c) | i) Write a note on maintenance of culverts. ii) Draw a neat sketch of a permanent way and mark its component parts. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Harbours |  |
| b) | Afflux and Scour |  |
| c) | Prestressed concrete bridges |  |
| d) | Widening of roads on curves |  |

***********