## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Electronics Engg./EC/E/E&EE

Subject: Electronic Instrumentation(4141) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | With the help of diagram explain operation of Piezo-electric Crystal. |  |
| b) | Why platinum is commonly used in RTD? What is PT-100? |  |
| c) | List some materials used in Thermocouple and RTD. What is Seeback effect? |  |
| d) | What is Signal conditioning? Draw the circuit diagram of a 3- opamp Instrumentation Amplifier? |  |
| e) | Draw a Seismic accelerometer and why is it used. |  |
| f) | Define i) Set point ii)Process Control iii) Controlled Variable. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Give 6 characteristics of a good transducer and 6 advantages of Electrical transducer over Mechanical transducer. |  |
| b) | Give construction & working of L.V.D.T using relevant diagrams. What is Residual voltage? |  |
| c) | With appropriate diagrams, write notes on: i) Unbonded strain gauge ii) Bonded strain gauge. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the working of 3-bit shaft encoder with neat diagrams. |  |
| b) | Explain the working of Stroboscope. |  |
| c) | How speed is measured using i) Toothed rotor ii) D.C. tachometer? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of diagram explain construction and operation of Turbine flow meter. |  |
| b) | With the help of diagram explain one Low pressure measurement technique. |  |
| c) | Explain following Linear velocity measurement technique: i) Moving magnet type ii) Moving coil type. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a block diagram, explain AC signal conditioning. |  |
| b) | With the help of block diagram explain single channel DAS. |  |
| c) | Explain the working of a Data logger with a block diagram. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | In detail explain: i) Two position controller mode ii) Multiposition controller mode |  |
| b) | With block diagram explain Direct Digital Control OR Computer Supervisory Control. |  |
| c) | Short notes on any two: i) 2 process parameters ii) Process control loop iii) Final control operation iv) Nozzle flapper system. |  |

***********