## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Electronics Engg./EC

Subject: Programmable Logic Controller(EN611) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Classify PLCs based on their I/O capacity and structure. |  |
| b) | Draw and describe any six standard IEC 1131-3 symbols. |  |
| c) | Draw and describe any two data comparison Ladder Diagram instructions. |  |
| d) | Explain the amount of time delay generated by drawing an example Ladder Diagram with cascaded timers. |  |
| e) | Explain the concept of battery-backed relays in PLCs. |  |
| f) | Write a brief overview of PLC system documentation. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define a PLC. Describe the structure of a PLC with the help of a neat block diagram. |  |
| b) | Describe using a diagram, sinking input and sinking output in a PLC. |  |
| c) | Describe eight advantages of a PLC. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the ladder Diagram for the following Boolean expression: A+B⋅A+CD Also draw its equivalent Functional Block Diagram. |  |
| b) | Explain the following PLC Ladder Diagram instructions with an example rung for each: - Examine if open - Latching - Output energize |  |
| c) | Explain the PLC Ladder Diagram instructions for the following, with an example Ladder diagram for each: - Subroutine - Data movement |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the function block of an up-counter used in PLC Ladder programming and explain its parameters. Distinguish between timers and counters in PLCs. |  |
| b) | Draw a ladder diagram for a bottle-filling plant process. The START button starts a conveyor belt, that carries only good bottles to be filled for 50 seconds each and then capped. Six filled and capped bottles are packed together and the process repeats. The entire process can be stopped by pressing the STOP push button. |  |
| c) | (i) Draw a Ladder Diagram to turn a lamp ON after 20 seconds of pressing the START push button. A STOP push button stops this process. (ii) Explain input, output and program scan in a PLC. |  |
|  |  | …….2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of an example ladder diagram explain the Bit Shift Right instruction. |  |
| b) | Write a note on each of the following: - Internal relays and their application - Master Control Relay |  |
| c) | Draw Ladder Diagrams to represent the following logic functions: - OR - EXOR - NAND |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the following fault-detection techniques in a PLC: - Replication - Last Output Set |  |
| b) | Describe the procedure for commissioning a PLC in an automation system. |  |
| c) | (i) Describe safety features provided in a PLC and the need for safety PLCs. (ii) With the help of a diagram, describe a triac output of a PLC. Distinguish between relay, transistor and triac outputs of PLCs. |  |

***********