## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Mechanical Engg.

Subject: Fluid Power Control(ME627) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State any three advantages of fluid power. |  |
| b) | What is meant by positive and non-positive displacement pumps? |  |
| c) | State the functions of an accumulator. |  |
| d) | What is single acting cylinder? How is it retracted? |  |
| e) | Under what conditions flexible hoses would be used in hydraulic systems? |  |
| f) | State any three safety measures to be observed while working with a hydraulic system. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Compare fluid power transmission with electrical and mechanical power transmission. (Any three points) |  |
| b) | Describe the construction and working of a Radial piston pump. |  |
| c) | Describe the following pipe fittings giving neat sketches: (i) Elbow ii) Reducer |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the construction and working of a poppet type 3/2 way valve and give its symbol. |  |
| b) | State any four methods of actuation of Direction control valves and give their respective symbols. |  |
| c) | What is the function of a pressure relief valve? Describe the construction and working of a simple relief valve. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Illustrate any three types of cylinder mounting giving neat sketches. |  |
| b) | A pump is supplying 80 liters/min of oil to a 50mm diameter double acting cylinder. If the load acting during extension stroke is 5000N, and diameter of piston rod is 25mm, find- - Hydraulic pressure in the system - Force during return stroke - Speed during extension stroke |  |
| c) | Draw a hydraulic circuit to illustrate meter-in flow control and explain. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and explain counter balance hydraulic circuit. |  |
| b) | Explain an ‘ÓR’ gate and give a pneumatic circuit for it. |  |
| c) | Draw a circuit using Quick exhaust valve and explain its working. |  |
| Q.No.6. | Write notes on any three: | 3 x 4 = 12 |
| a) | Hydraulic power unit |  |
| b) | Regenerative circuit |  |
| c) | Two handed safety circuit |  |
| d) | Maintenance of a pneumatic system |  |

***********