## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Electrical Engg./E&EE

Subject: Switchgear & Protection(EE501)/(4110) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List the types of symmetrical faults & unsymmetrical faults on power system. |  |
| b) | What is a fuse? State its one advantage & one disadvantage. |  |
| c) | List desirable characteristics of fuse element. |  |
| d) | Draw a diagram showing restriking voltage & recovery voltage. |  |
| e) | State any six advantages of static relay. |  |
| f) | For relays define: i) Reset ii) Pick up. |  |
| g) | Briefly explain difference between Main protection and Back-up protection. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is current limiting reactor? State and briefly explain their possible locations. |  |
| b) | State any six differences between HRC fuse and rewirable fuse. |  |
| c) | A three phase, 11KV, 5000KVA generator has reactance of 20%. It is connected to a 3000 KVA transformer having 5% leakage reactance and a ratio of 11/33 KV. The 33 KV side is connected to a transmission line. A three phase fault occurs at the other end of transmission line. The series reactance between the faulted point and transformer is 30. Calculate the fault KVA & fault current. |  |
| d) | With a neat diagram explain Earth fault & Overcurrent protection of an alternator. (only for students of old Scheme) |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State and briefly explain two methods of arc extinction. |  |
| b) | With neat diagram explain construction and working of ELCB. |  |
| c) | With neat diagram explain construction and working of MCB. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat diagram explain working of Air circuit breaker. |  |
| b) | With a neat diagram explain working of puffer piston type of SF6 circuit breaker. |  |
| c) | Explain different arc control devices used in oil circuit breaker using simple diagrams. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain working of any two attraction armature type of relays. |  |
| b) | With a block diagram explain static overcurrent relay. |  |
| c) | Explain the concept of i) Bi-metallic relay ii) Biased differential protection. | ……2/- |
|  | -2- |  |
| d) | With a neat diagram explain unit protection for generator and transformer. (only for students of old Scheme) |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram explain current graded protection. State its advantages and disadvantages. |  |
| b) | With a neat diagram explain earth leakage protection of bus bars. |  |
| c) | Explain how induction motor can be protected from single phasing and low voltage. |  |

***********