## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Electronics Engg./EC

Subject: Communication Systems-II(4144) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is super refraction? Explain in brief. |  |
| b) | Define i) Critical frequency ii) Polarization. |  |
| c) | What is characteristic impedance of transmission line? Explain in brief. |  |
| d) | Define the following terms: - Power gain ii) Beamwidth |  |
| e) | Compare resonant antenna with non-resonant antenna. |  |
| f) | State and explain Nyquist sampling theorem. |  |
| g) | What is the need for multiplexing in communication? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat diagram of the ionosphere and explain how it helps in sky wave propagation. |  |
| b) | i) Define the following terms: 1) VSWR 2) Reflection coefficient. ii) Explain how a quarterwave line can be used for impedance matching. |  |
| c) | Draw a neat diagram and explain the working of a 5 element Yagi Uda antenna. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain why a cut-off wavelength exists for a rectangular waveguide. Find out the cut-off wavelength for the dominant mode in a waveguide having wall separation of 3 cms. |  |
| b) | Draw a neat block diagram and explain the working of a PPM modulator. |  |
| c) | Draw a neat diagram of a communication system based on PCM and explain the function of each block. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the following terms: - Granular noise ii) Slope overload |  |
| b) | With the help of block diagram, explain the working of a communication system based on adaptive delta modulation. |  |
| c) | Draw a neat block diagram and explain the working of an ASK demodulator. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat block diagram and explain the working of a QPSK demodulator. |  |
| b) | Draw a neat block diagram of a communication system using TDM and explain its working. |  |
| c) | Briefly explain the following: i) Antialiasing filter ii) FSK. |  |
| Q.No.6. | Writ short notes on any four: | 4 x 3 = 12 |
| a) | Ground wave propagation |  |
| b) | Broadside array v/s Endfire array |  |
| c) | Group velocity v/s Phase velocity |  |
| d) | FDM |  |
| e) | Horn antenna |  |

***********