## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Architectural Engg.

Subject: Theory of Structures(AE405)/Structures-I(4537) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define force. What are the characteristics of force? |  |
| b) | Explain moment of force with example. |  |
| c) | Explain effects of force on a body. |  |
| d) | State parallelogram law of forces. Give example. |  |
| e) | What are grades of concrete? Where they are used? |  |
| f) | What are the factors affecting concrete mix design? |  |
| g) | What properties should the reinforcement posses as structural material in reinforced cement concrete? |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Find the resultant in magnitude and direction of the following concurrence forces: i) 250N acting 90° ii) 300N acting at 120° iii) 350N acting 220° iv) 400N acting at 340°. |  |
| b) | Find the resultant in magnitude and direction using Parallelogram law of the following forces: i) 200N acting at 45° and ii) 400N acting at 145° from positive X-axis. |  |
| c) | Find the resultant in magnitude and direction of following force system: i) 5KN acting due North –East ii) 7KN acting at 30° North of west (iii) 9KN acting due south. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw S.F.D and B.M.D for a cantilever of span 5m carrying uniformly distributed load of 40KN/m over the entire span. |  |
| b) | A simply supported beam of 8m span carries two point loads 2KN and 5KN at 2m from each support. Draw S.F.D and B.M.D. |  |
| c) | Draw shear force diagram and bending moment diagram for a beam of span ‘L’ m carrying u.d.l. of W N/m over entire span when it is i) Simply supported on both ends ii) When it is cantilevered with fixed support at one end and free at other. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the centroid of an ‘I’ section whose top flange is 100m x 20mm, bottom flange is 200mm x 30mm and web is 20mm x 150mm. |  |
| b) | Explain what are load bearing structures with reference to special consideration in seismic zone. |  |
| c) | What are the structural elements of framed structure? |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are structural and non-structural cracks? |  |
| b) | What are the causes of cracks in masonry structure? |  |
| c) | How errors in design and detailing cause cracks? |  |
|  |  | …….2/- |
|  | -2- |  |
| Q.No.6. | Write short note on any three: | 3 x 4 = 12 |
| a) | One way and two way slabs |  |
| b) | Corbelling in masonry |  |
| c) | Short columns and long columns |  |
| d) | Domes and Arches |  |
| e) | Seismic effects on framed structures |  |

***********