## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Computer Engg.

Subject: Microprocessors & Applications(CO401) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the following 8086 instructions: - RCR (ii) NOT (iii) PUSH |  |
| b) | Discuss any 3 conditional flags in 8086. |  |
| c) | Explain the role of assembler in assembly language program development. |  |
| d) | List the important features of traffic light controller with respect to 8086 interfacing. |  |
| e) | Explain the following pins/signals in 8086: - MN/MX (ii) NMI (iii) M/IO |  |
| f) | Explain with an example: i) Immediate Addressing Mode ii) Implied Addressing Mode |  |
| g) | Explain the role of segment registers in 8086. |  |
| h) | List the pins that are used in minimum mode. Explain any two. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the functional block diagram of 8085 processor. Explain each block in detail. |  |
| b) | Explain the role of Execution Unit and Bus Interface unit in 8086. Also discuss on the registers used in each unit. |  |
| c) | Draw the 8086 flag register. Explain the control flags in detail. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write an 8086 assembly language program to compare whether two given strings are equal or not. |  |
| b) | Explain with suitable examples any four addressing modes in detail. |  |
| c) | Describe the operation that 8086 will perform when it executes each of following instruction: i) MUL BX ii) SBB BH, AL iii) MOV AX, [CX] iv) SAL DX,1 v) OUT 4Fh, AL vi) LODS SOURCE-STRING |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain reentrant and recursive procedures with neat diagrams. |  |
| b) | Explain the role of Stack pointer in 8086. Also discuss the PUSH and POP Stack operations in detail. |  |
| c) | Explain the various assembler directives with suitable examples. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat timing diagram, explain 8086 Read machine cycle. |  |
| b) | List the steps and explain in detail how 8086 responds to an Interrupt. |  |
| c) | Explain 8086 in maximum mode. Explain the functions of pins common to minimum and maximum mode. | …..2/- |
|  | -2- |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the various architectural features of Pentium processor. |  |
| b) | Explain with neat diagram, interfacing of 8086 with Digital to Analog Converter. |  |
| c) | With neat diagrams, explain interfacing of 8086 with Memory array and ROM/EPROM. |  |

***********