## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Mining Engg.

Subject: Surface Mining(MN401)/(4298) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Write classification of surface mining method. |  |
| b) | Enumerate methods of Brush cleaning. |  |
| c) | Give the classification of excavation and loading machines. |  |
| d) | State suitability conditions of dumper transportation system. |  |
| e) | Explain types of waste dump. |  |
| f) | Enumerate factors affecting dump site selection. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define bench height and explain factors affecting bench height. |  |
| b) | Explain opencast mining feasibility study. |  |
| c) | i) Enumerate stages in surface mining. ii) List out methods of drilling. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe methods of grubbing. |  |
| b) | Enumerate components of ripper and state their functions. |  |
| c) | Write formula for power shovel capacity and state factors affecting capacity. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Calculate production of a power shovel under following conditions: Number of shift – 2 Total duration of shift – 8 Effective hours/shift – 7 Shovel bucket capacity = 3m3 Swell factor = 1.2 Bulk weight of material = 2.5 tonnes/m3 Fill factor = 0.8 Cycle time = 20 seconds |  |
| b) | With sketch explain types of faces and cut by Bucket wheel excavator. |  |
| c) | Compare power shovel with drageline. |  |
| Q.No.5. | Answer the following Questions: | 2 x 6 = 12 |
| a) a) | A shovel of 4.6m3 capacity is employed in a opencast mine project and particulars are as follows: Number of pass – 5 Bulk density of material = 1.7 tonnes/m3 Bucket fill factor = 0.8 Material swell factor = 1.2 Shovel cycle time = 20seconds Haul distance = 2.5Km Discharging time of dumper = 45 seconds Spotting time = 30 seconds Calculate types and number of dumper required. ### OR -2- Write a note on ‘Selection of belt conveyor and material used in belt’. | …..2/- |
| b) | Describe suitability condition, advantages and disadvantages of dumper transportation system. |  |
| Q.No.6. | Write short notes on: | 3 x 4 = 12 |
| a) | Dump construction by Bulldozer |  |
| b) | Blast design |  |
| c) | Steps in drilling and blasting |  |

***********