## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Process Control(4203) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are the standard signals used in process control system? |  |
| b) | What are the process specifications in discrete state control? |  |
| c) | Explain: Process load, Process equation. |  |
| d) | Explain implementation of any one controller mode using pneumatic controller. |  |
| e) | Explain i) Simple alarm in process control ii) Fixed logger. |  |
| f) | What is meant by process loop tuning? |  |
| g) | Describe constructional details of control valve. Define valve Cv. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain concept of control system and develop a block diagram showing elements of process control system. |  |
| b) | Explain concept of discrete state process control. Specify its characteristics. What are discrete state variables? |  |
| c) | Explain concept of ladder diagram. With help of example, develop a ladder diagram showing various elements. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain Programmable controller, showing elements with typical input and output module. |  |
| b) | Explain i) Process lag ii) Self regulation iii) Variable range. |  |
| c) | What is meant by mode of controller operation? Explain the classification of controller modes of operations. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain operation of i) any one discontinuous controller mode ii) any one continuous controller mode. |  |
| b) | Explain implementation of following modes using Opamps: - PI ii) PD |  |
| c) | Explain implementation of following modes using Pneumatic controllers: - P ii) PID |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain DAS with block diagram. |  |
| b) | Explain computer based controller. |  |
| c) | Write notes on i) Cascade control ii) Optimum control. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a note on i) Control system quality ii) Loop disturbances. |  |
| b) | With help of block diagram, explain elements of final control operation. |  |
| c) | Explain: i) any one actuator operation ii) any one control element. |  |

***********