## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Electrical Engg./E&EE

Subject: Utilisation of Electrical Energy(EE403)/(4111) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List different modes of heat transfer. Which is the efficient mode of heat transfer among all. Explain why. |  |
| b) | Explain briefly Infrared heating. |  |
| c) | Give three points of comparison between AC welding and DC welding. |  |
| d) | Explain the concept of arc formation. |  |
| e) | Explain the concept of stroboscopic effect. |  |
| f) | Give requirements of fraction motors. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List and explain advantages of electric heating over other methods of heating. |  |
| b) | Give classification of electric heating methods and explain each class briefly. |  |
| c) | A 15KW, 220 Volts, single phase resistance oven uses Nickel-Chrome wire for its heating element. If wire temperature is not to exceed 1000ºC and temperature of charge is to be 600ºC, calculate the diameter and length of wire. Given radiating efficiency = 0.6, emissivity = 0.9, specific resistance of Nickel-Chrome = 1.016 x 10-6Ωm. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List different core type induction furnaces. Explain any one type with a neat diagram. |  |
| b) | Explain high frequency eddy current heating. |  |
| c) | Give advantages and applications of dielectric heating. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain any four methods of current control in AC welding. |  |
| b) | With a neat diagram explain projection welding. Give any two advantages of projection welding over spot welding. |  |
| c) | With neat diagram explain any 2 heat control methods employed for resistance heating equipments. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and explain briefly Speed-Time curve for: i) Urban services of electric traction ii) Suburban services of electric traction. |  |
| b) | List different current collectors used in overhead system of traction. Explain current collection by any one method with a neat diagram. |  |
| c) | With a neat diagram explain bridge transition method. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define the following terms: i) Luminance ii) Reflection factor iii) Absorption factor. |  |
| b) | Explain with a neat diagram, construction and working of low pressure mercury vapour lamp. |  |
| c) | Explain the principle of production of light in i) Gas discharge lamp ii) Filament lamp. |  |

***********