## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Mechanical Engg.

Subject: Principles of Airconditioning(5047) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Psychrometric Charts and Steam Tables is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the following Psychrometric properties: - Dew point temperature - Specific humidity - Wet bulb depression |  |
| b) | Define the following: - By-pass factor of coil - Sensible heat factor - Apparatus dew point |  |
| c) | What is infiltration air load? What are the sources? |  |
| d) | What are the basic types of supply air outlets used in distribution of air to an airconditioned space? |  |
| e) | List the types of insulating materials used in Refrigeration and airconditioning systems. |  |
| f) | What are the methods employed for controlling room conditions at partial load? |  |
| g) | What is meant by Economical thickness of Insulation? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A sample of moist air is at 30C DBT and 60% RH under a total pressure of 1.013 bar. Determine i) Dew point temperature ii) Specific humidity iii) Specific enthalpy of the moist air using steam table and empirical formulae. |  |
| b) | With the help of sketch explain the working principle of ‘Aspirating type Psychrometer’. |  |
| c) | Ä rectangular duct of size 40m x 30m is used to carry conditioned air. Determine the equivalent size of circular duct, if the i) Velocity in both the ducts are same ii) Quantity of air supply is same. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the following psychrometric processes and represent the same in psychrometric chart: i) Cooling and humidifying at constant WBT. ii) Heating and dehumidifying |  |
| b) | With the help of neat sketch, explain ‘Winter airconditioning system’ for cold and dry location. Represent the process in psychrometric chart. |  |
| c) | Atmospheric air at 40C DBT and 60% RH is passed over a cooling and dehumidifying coil at the rate of 450m3/min. If the air leaving the coil has 20C DBT and 15C WBT, find the following: i) Capacity of the cooling coil in TR ii) Quantity of water vapour removed from the air in Kg/h |  |
|  |  | …..2/- |
|  | -2- |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat labelled diagram of Central Airconditioning ‘DX’ System. |  |
| b) | Differentiate between ‘Áll air system’ and ‘Áll Water system’ of airconditioning. |  |
| c) | Discuss the special features of Airconditioning systems used in Computer centres. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain how the following heat sources are accounted for cooling load estimation: - Occupancy load ii) Infiltration load iii) solar heat gain |  |
| b) | An air-conditioned space is maintained at 24C DBT and 50% RH, when the outdoor conditions are 35C DBT and 28C WBT. The space has a sensible heat gain of 25 KW and the air is supplied to the space at 10C saturated condition. Determine i) the mass flow rate of air supplied in Kg/s ii) Latent heat load in the room iii) Cooling load of the A/C plant in TR if 20% of total mass of the air supplied to the space is fresh air and remaining is recirculated. |  |
| c) | With a neat sketch, explain the construction and working of any one type of humidifier. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch, explain the working principle of Electronic air filters. |  |
| b) | With a neat sketch, explain the two position electric control system. |  |
| c) | Explain Face and By-pass control system. |  |

***********