## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Computer Engg.

Subject: Microprocessors-III(8086) [5178] [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain significance of following pins of 8086: - ALE ii) INTR iii) AD15 – AD0 |  |
| b) | List the functions of any three different conditional flags in 8086. |  |
| c) | Explain use of following 8086 instructions: - JZ ii) CMP iii) LOOP |  |
| d) | Differentiate between Macros and Procedures. |  |
| e) | Explain in brief various types of 8086 interrupts. |  |
| f) | List various 8086 processor control instructions and explain any two. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the role of 8086 in minimum mode. Also list and explain various pins used in minimum mode. |  |
| b) | Draw the block diagram of 8086 microprocessor and explain the role of its segment registers. |  |
| c) | Explain any six addressing modes of 8086. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain 8086 string instructions, with examples of each. |  |
| b) | Write a 8086 assembly language program to multiply two 8 bit hexadecimal numbers. |  |
| c) | Explain the purpose of following 8086 instructions with examples: i) DEC ii) POP iii) OUT iv) CBW v) ROR vi) SUB. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss Re-entrant and Recursive procedures with examples. |  |
| b) | Explain in detail any six assembler directives of 8086 with examples. |  |
| c) | Discuss how interrupts are handled in 8086. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the block diagram of 8087 internal architecture and explain its any two data-types. |  |
| b) | Explain any 6 instructions of 8087 math co-processor. |  |
| c) | Explain with a neat timing diagram, 8086 Read Machine Cycle. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Port addressing modes of 8086 |  |
| b) | Assembly language program development tools |  |
| c) | Steps in troubleshooting 8086 based microcomputer |  |
| d) | Features of 80286 and Pentium microprocessors |  |

***********