## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Mechanical Engg.

Subject: Computer Aided Design & Manufacturing(ME613) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are the input devices more commonly employed for general graphics applications? |  |
| b) | Define CAD and CAM. |  |
| c) | What are free form surfaces in geometric modelling? |  |
| d) | What are end effectors? |  |
| e) | What is CIMS? |  |
| f) | What do you mean by host satellite CAD system? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | How do you classify the various geometric modelling systems based on their capabilities? |  |
| b) | What is meant by sweep? Discuss in detail the various types of sweep techniques available in 3D geometric construction. |  |
| c) | Explain the concept of basic Boolean operations used in solid modelling. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the difference between NC and CNC. |  |
| b) | What are the various types of motion control systems used in CNC? Explain any one system in brief with a neat sketch. |  |
| c) | Briefly explain the basics of designating the co-ordinate axes in CNC machine tools. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | For the component shown in fig.(1), write a part program on a machining centre equipped with the ISO controller. |  |
| b) | Referring to fig.(2), prepare a part programme using a CNC turning centre. |  |
| c) | What are the differences in part programming to be considered between CNC turning centres and CNC machining centres? |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the applications for which robots are generally used. |  |
| b) | What are the various types of motion control possible in robots? |  |
| c) | Describe the requirements of sensors in robots. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss the various levels of automation. |  |
| b) | Describe the factors responsible for the success of CIM applications in the present time. |  |
| c) | Explain the various types of layouts used in FMS design. |  |
|  |  | …..2/- |