## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Architectural Engg.

Subject: Building Construction-III(AE303) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | When is eccentrically loaded shallow foundation used? |  |
| b) | List 2 requirements of a good scaffolding. |  |
| c) | Define spanning for a RCC staircase. |  |
| d) | What purpose does mullion serve in a timber window? |  |
| e) | List any 4 types of trusses other than King-post truss and Queen post truss. |  |
| f) | Why is the timber frame grooved on both sides of the inner edges and rebated on the outer edge? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw sketch plan and section each of i) Cellular construction Raft ii) Two way beam and slab type raft. |  |
| b) | Explain any 3 points which need to be considered while providing foundation on sloping ground. |  |
| c) | Write short notes on: i) Spread footing ii) Raft foundation. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Differentiate between construction joints and expansion joints. Give 3 points of each. |  |
| b) | Write short notes on: i) Curing of concrete ii) Compaction of concrete |  |
| c) | Explain with a sketch the provision of shuttering for beam and slab floor in R.C.C. construction. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat labelled cross-section of a timber staircase. |  |
| b) | Differentiate in sketch plan between a dog-legged stairs and a bifurcated staircase. |  |
| c) | Explain any one: - Types of stringers in a timber staircase - Types of stone staircases |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat labelled elevation of a timber glazed casement window. Explain any 3 terminologies used. |  |
| b) | Draw the plan and elevation of a 3 shutter typical metal window. |  |
| c) | Explain the method of fixing of timber door frame. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat labelled diagram of a Queen post truss. |  |
| b) | Differentiate between Double or Purlin roof and Trussed roof. |  |
| c) | Write short notes on: - Wall plate - Eaves board - Ridge beam |  |

***********