## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Fabrication of Boilers & Pressure Vessels(FE703)/(5341)

[Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is Dead Load? |  |
| b) | Write a note on bolting materials. |  |
| c) | How is rolling of truncated cone carried out? |  |
| d) | How is lubrication carried out during tube bending? |  |
| e) | What are thermal stresses? |  |
| f) | What are the limitations of Press braking? (only for students of revised scheme) |  |
| g) | What are anchor bolts? (only for students of old scheme) |  |
| h) | Write a note on dilation of Pressure vessel. (only for students of old scheme) |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | How are stresses analysed in a thick cylindrical shell subjected to internal pressure? |  |
| b) | What are discontinuity stresses in pressure vessels? How can these be controlled? |  |
| c) | Explain the pyramid type rolling machine with three rolls with adjustable bottom rollers with a neat sketch. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain spinning lathes in brief. |  |
| b) | How is inspection carried out during welding in construction? |  |
| c) | Describe the stresses in Bi-metallic joints with a neat sketch. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the different types of mandrels used for tube bending with a neat sketch. |  |
| b) | State the reasons for stiffening. Explain the basic principle of stiffening. |  |
| c) | How does a pressure vessel buckle under external load? What is fatigue failure? |  |
| Q.No.5. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Selection of steel for hydrogen service |  |
| b) | Bending thin wall tubes |  |
| c) | Selection of material for non-corrosive service |  |
| d) | Hydro test and procedure |  |
| e) | Hot bending |  |
| f) | Importance of IBR code (only for students of old scheme) |  |
| f) | Hand operated fly press (only for students of revised scheme) |  |
|  |  | …….2/- |
|  | -2- |  |
|  | only for students of old scheme: |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | How are boilers classified? |  |
| b) | Explain the following terms: - Percentage elongation iii) Poisson ratio - Elastic Limit iv) Toughness |  |
| c) | Write short notes on following: - Deformation and stress in flanges - Stacked plates and built up plates |  |
|  | only for students of revised scheme: |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Give brief description of operations carried out on a press. |  |
| b) | What is spring back? Explain the method to compensate spring back with a neat sketch. |  |
| c) | State the principle of press brake. Explain the types of press brakes. |  |

***********