## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Mechanical Engg./A/SB

Subject: Strength of Materials(ME401)/(4032) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define stress, strain and state the relationship between the two. |  |
| b) | What are principal planes? How do the stresses occur on principal planes? |  |
| c) | State and explain the parallel axis theorem. |  |
| d) | What are the assumptions made in the theory of pure torsion? |  |
| e) | State bending equation and the terms used in bending equation. |  |
| f) | Explain Lateral strain, Longitudinal strain and Poisson’s ratio. |  |
|  | Only for students of Old scheme: |  |
| g) | Derive the equation for Direct and Bending stress. |  |
| h) | Define thick and thin cylinders. Explain the difference between them with sketches. |  |
| i) | Determine the equation for the strain energy stored in a body when the load is suddenly applied. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the stress strain diagram with a neat sketch. |  |
| b) | A rod of steel is 30m long and at a temperature of 25C. Find the free expansion of the road when the temperature is raised to 70C. Find the temperature stresses produced when the rod is permitted to expand by 6mm. Assume  = 12x10-6/C and E= 2x105N/mm2. |  |
| c) | The principal stresses at a point in a material are 200N/mm2 (Tensile) and 100N/mm2 (Compressive). Determine the resultant stress in magnitude and direction on a plane inclined at 60 to the axis of the major principal stress. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A cantilever beam AB 3m long carries point loads of 3KN, 4KN and 5KN at distances 1m, 2m, 3m from fixed end. Calculate the Shear Force and Bending Moment. Also draw the diagram. |  |
| b) | A beam 6m long is simply supports at its ends and carries point loads of 140KN, 60KN and 80KN at distances 0.5m, 2.5m and 3.5m from the left support. Calculate the Shear Force and Bending Moment. Also draw the diagram. |  |
| c) | A weight of 250N is dropped on a collar attached to the end of a vertical bar 1.5m long and 25mm in diameter from a height of 200mm. Calculate the maximum instantaneous elongation and the stress produced in the section of the bar. E=2x105N/mm2 (Only for students of Old scheme) -2- | …..2/- |
| c) | A simply supported beam 6m in span carries two point loads of 200N each at 2m from either side of the support. It also carries an u.d.l. of 250N/mm inbetween the two point loads. Draw S.F.D and B.M.D. (Only for students of Revised scheme) |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the Moment of Inertia about the centroidal XX axis of a ‘L’ section where the vertical leg is 20mm x 100mm (Overall depth=100mm) and extended horizontal leg is 60mm x 20mm (Overall width = 80mm). |  |
| b) | A rectangular cantilever beam is having 80mm width and 120mm depth. If the cantilever is subjected to a point load of 60KN at its free end and the bending stress is not to exceed 40N/mm2, find the span of the beam. |  |
| c) | A rectangular beam 60mm wide and 180mm depth is simply supported at a span of 4m. If the beam is subjected to a UDL of 4.5KN/m, find the maximum bending stress induced in the beam. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A masonry column is 600mm x 600mm in section and carries a point load of 100KN acting on the XX centroidal axis at an eccentricity of 80mm from YY axis. Find the maximum and minimum stresses in the section. |  |
| b) | A solid shaft is of 100mm diameter. It transmits 150KW at 200rpm. Find the maximum intensity of shear stress induced and the angle of twist for a length of 6m. Take C= 8x104N/mm2. |  |
| c) | A solid shaft of 600mm diameter is running at 160 rpm. Find the power, which the shaft can transmit, if the permissible shear stress is 80N/mm2 and the maximum torque is likely to exceed the mean torque by 30%. |  |
|  | Only for students of Old scheme: |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A cylindrical shell of 2m diameter is made up of 20mm thick plates. Find the circumferential and longitudinal stress if the shell is subjected to an internal pressure of 3N/mm2. |  |
| b) | A strut 2.5m long is 60mm in diameter. One end is fixed and other end is hinged. Find the safe compressive load using Euler’s formula. Use factor of Safety as 3.5 and E = 2x105N/mm2. |  |
| c) | A simply supported beam of span 6m is subjected to a UDL over the entire span. If the deflection at the centre of the beam is not to exceed 4mm, find the load. E=2x105N/mm2 and I = 300x106 mm4 |  |
|  | Only for students of Revised scheme: |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the moment of Inertia of inverted ‘T’ section whose flange and web thickness is 30mm, overall depth is 250mm and overall width is 150mm about centroidal X-X axis and centroidal Y-Y axis. |  |
| b) | Find the principal planes and principal stresses at a certain point in a strained material the intensities of normal stress on two planes at right angles to each other 20N/mm2 and 10N/mm2 both tensile alongwith shear stress of 10N/mm2. |  |
| c) | Explain with neat sketches concept of shear stress and strain and define the modulus of rigidity. State the relationship between the Young’s modulus and Modulus of rigidity. |  |

***********